Basavarajeswari group of Institutions

BALLARI INSTITUTE OF TECHNOLOGY & MANAGEMENT

(Autonomous Institute under VTU, Belagavi)

(Recognized by Govt. of Karnataka, approved by AICTE, New Delhi & Affiliated to Visvesvaraya Technological University, Belagavi)

"JnanaGangotri" Campus, No.873/2, Ballari-Hospet Road, Allipur, Ballari-583 104 (Karnataka State)

2022-2023

DEPARTMENT OF COMPUTER SCIENCE AND ENGINEERING

A Project Report On

“DEFAKE AI: A Machine Learning Based Approach to Audio-Video Detection”

A dissertation submitted to the Department of Computer Science and Engineering of Visvesvaraya Technological University in partial fulfilment for the award of the Degree of Bachelor of Engineering.

Submitted By

RUHE TARANNUM	3BR22CS141

SADIYA TAHSEEN	3BR22CS143

SHAHID AHMED	3BR22CS150

SHAIK FARHATH AMREEN	3BR22CS151

Under the Guidance of

Mr. Mohammed Shafiulla

Assistant Professor Dept of CSE BITM, Ballari.

Visvesvaraya Technological University

Belagavi, Karnataka

2025-2026

Basavarajeswari group of Institutions

BALLARI INSTITUTE OF TECHNOLOGY & MANAGEMENT

(Autonomous Institute under VTU, Belagavi)

(Recognized by Govt. of Karnataka, approved by AICTE, New Delhi & Affiliated to Visvesvaraya Technological University, Belagavi)

"JnanaGangotri" Campus, No.873/2, Ballari-Hospet Road, Allipur, Ballari-583 104 (Karnataka State)

2022-2023

DEPARTMENT OF COMPUTER SCIENCE AND ENGINEERING

CERTIFICATE

This is to certify that the project work entitled “DEFAKE AI: A Machine Learning Based Approach to Audio-Video Detection” is a bonafide work carried out by RUHE TARANNUM, SADIYA TAHSEEN, SHAHID AHMED, SHAIK FARHATH AMREEN in partial fulfilment for the award of degree of Bachelor of Engineering in Computer Science and Engineering in the Visvesvaraya Technological University, Belagavi during the year 2025-2026. It is certified that all corrections and suggestions indicated for internal assessment have been incorporated in the report deposited in the library. The project has been approved as it satisfies the academic requirements in respect of project work prescribed for Bachelor of Engineering degree.

| Signature of Project Guide | Signature of HOD | Signature of Principal |
| --- | --- | --- |
| Mr. Mohammed Shafiulla | Dr. R. N. Kulkarni | Dr. Yadavalli Basavaraj |

External Viva

Name of the Examiner:	Signature with Date 1.

2.

ACKNOWLEDGEMENT

We express our warm and profound sense of gratitude to all the eminent faculties who inspired, guided and supported us in accomplishing our project work.

We are deeply indebted to Mr. MOHAMMED SHAFIULLA, Department of CSE, our guide on this project & coordinator Mr. CHANDRASHEKAR V, on this project, for consistently providing us with the required guidance to help us in the timely and successful completion of this project. In spite of his extremely busy schedules in Department, he was always available to share with us his deep insights, wide knowledge and extensive experience.

We are much obliged to our honourable Head of the Department Dr. R. N. Kulkarni for understanding the capability in us to complete the project successfully and, we are thankful to him for encouraging us and being motivation and inspiration for us.

We extend our warm thanks to our Principal Dr. Yadavalli Basavaraj for giving us the moral support and providing us all kinds of facilities.

We also thank all the faculty members of our department for being with us when needed. Last but not the least we also thank our beloved friends who were there with us all the time supporting, providing assistance and giving us enough strength to successfully complete our project and also proving the popular saying, “A Friend in need is a Friend indeed” to be true.

NAME	    USN

RUHE TARANNUM	     3BR22CS141

SADIYA TAHSEEN	     3BR22CS143

SHAHID AHMED	     3BR22CS150

SHAIK FARHATH AMREEN	     3BR22CS151

ABSTRACT

The Interactive Vehicle Accident Response System (IVARS) is an innovative web-based platform designed to revolutionize emergency response to vehicular accidents. This project addresses the critical challenge of delayed emergency response times by providing a real-time, GPS-enabled incident reporting system that connects accident reporters with emergency responders efficiently.

IVARS leverages modern web technologies including React, TypeScript, Node.js, Express.js, and MongoDB to create a scalable, secure platform. The system integrates Google Maps API for precise location tracking, Cloudinary for image storage, and SendGrid for automated notifications. Key features include real-time incident mapping, role-based access control (Citizen, Responder, Admin), image upload capabilities, automated responder alerts based on proximity, and comprehensive analytics dashboards.

The system successfully reduces incident reporting time from 5-10 minutes (traditional phone calls) to under 3 minutes through a user-friendly web interface. GPS coordinates eliminate location ambiguity, while uploaded images provide responders with crucial visual context before arrival. Testing with 25 users achieved a 92% ease-of-use rating and demonstrated significant improvements in emergency response coordination.

IVARS represents a significant advancement in emergency management technology, with potential applications beyond vehicular accidents including medical emergencies, natural disasters, and civic issue reporting.

| Table of Contents |  |  |
| --- | --- | --- |
| Chapter No | Chapter Name | Page No |
|  | Acknowledgement |  |
|  | Abstract |  |
| 1 | Introduction | 1 |
| 2 | Literature Survey | 6 |
| 3 | Requirements Specifications | 17 |
| 4 | System Design | 20 |
| 5 | Implementation | 26 |
| 6 | System Testing | 36 |
| 7 | Results and discussion | 39 |
| 8 | Conclusion and future scope | 44 |
| 9 | References | 45 |
| List of Tables |  |  |
| Table No | Table Name | Page No |
| 6.1 | Unit Testing | 37 |
| List of Figures |  |  |
| Figure No | Figure Name | Page No |
| 7.1 | Homepage and User Interface | 39 |
| 7.2 | Uploaded Image | 39 |
| 7.3 | Image Detection Results. | 40 |
| 7.4 | Audio Uploaded. | 41 |
| 7.5 | Audio Detection Results. | 41 |
| 7.6 | Video Uploaded. | 42 |
| 7.7 | Video Detection Results. | 42 |
| 7.8.1 | Test Results. | 43 |
| 7.8.1 | Test Results. | 43 |
| 7.10 | Issue token page (Admin) | 47 |

INTRODUCTION

The Interactive Vehicle Accident Response System (IVARS) is a comprehensive web-based platform designed to streamline accident reporting and emergency response coordination. In an era of increasing vehicular traffic and road accidents, IVARS provides a real-time solution enabling faster emergency response.

The system leverages modern web technologies including React, TypeScript, Node.js, Express, and MongoDB to create a robust, scalable platform. It incorporates Google Maps API for precise location tracking, Cloudinary for secure image storage, and SendGrid for instant email notifications. IVARS supports multiple user roles including citizens, emergency responders, and administrators, each with specific access controls and functionalities.

Key features include real-time incident mapping, GPS-based location tracking, image upload capabilities, automated responder notifications based on proximity, role-based access control, and comprehensive analytics dashboards. The system aims to reduce emergency response times, improve coordination between civilians and responders, and maintain accurate records of all reported incidents.

According to WHO, approximately 1.35 million people die annually from road traffic accidents. Many fatalities result from delayed emergency response. IVARS addresses this critical problem by making accident reporting instant, accurate, and information-rich, potentially saving countless lives.

  - OVERVIEW

With the rapid advancement of artificial intelligence and digital media technologies, the manipulation of visual and audio content has become increasingly sophisticated, giving rise to a new and alarming phenomenon known as deepfakes. Deepfakes are AI-generated synthetic media designed to imitate real individuals or events, often used with malicious intent to spread misinformation, manipulate public opinion, or compromise personal and organizational integrity. In today’s interconnected world, where digital content is the primary medium for communication and information sharing, the need for a reliable system to verify content authenticity has become more critical than ever. The Deepfake Detection System addresses this challenge by providing an intelligent, automated, and user-friendly platform capable of detecting manipulated media across images, videos, and audio. Built using deep learning and computer vision techniques, the system integrates TensorFlow/Keras, OpenCV, NumPy, PIL, Librosa, and  Streamlit  to analyze and classify media authenticity with precision. By leveraging a hybrid MobileNet–LSTM model for video and image detection and a CNN-based architecture for audio analysis, the project ensures high accuracy and efficiency in identifying synthetic alterations. This solution not only helps preserve digital truth but also supports law enforcement, media organizations, and individuals in combating misinformation, safeguarding reputation, and maintaining the integrity of digital communication.

  - EXISTING SYSTEM

**Traditional Emergency Response Methods**:

**1. Phone-Based Reporting (911/100/108)**:
- Requires verbal communication during high-stress situations
- Location description depends on caller's area knowledge
- No visual evidence provided
- Phone lines can be busy
- No tracking of incident resolution

**2. Highway Emergency Call Boxes**:
- Fixed locations only
- Requires walking to find nearest box
- Limited information conveyed
- Aging infrastructure

**3. Basic Mobile Apps**:
- Some regions have basic emergency apps
- Limited to GPS sharing without comprehensive details
- No responder dashboard integration
- Poor user experience

**Drawbacks**:
- Average 5-10 minute reporting time
- 30-40% calls have incorrect location information
- No severity-based prioritization
- No visual context for responders
- No transparency for reporters
- No systematic data collection
- Coordination issues between agencies
- Language barriers

These systems don't leverage modern smartphone capabilities (GPS, cameras, internet connectivity).

  - PROPOSED SYSTEM

IVARS proposes a comprehensive solution addressing all limitations of existing systems:

**Core Features**:

**1. Instant Incident Reporting**:
- User-friendly web form
- GPS auto-capture or manual map selection
- Multiple image upload (up to 5 images)
- Severity level selection
- Vehicle and witness information
- Unique report ID generation

**2. Real-Time Map Visualization**:
- Interactive Google Maps display
- Color-coded markers by severity
- Popup details on marker click
- Filter by status and severity
- Real-time updates

**3. Smart Responder Notification**:
- Automated email to nearby responders
- Distance calculation from incident
- Complete incident details
- Image attachments
- One-click navigation links

**4. Role-Based Access**:
- **Citizens**: Report incidents, view map, track their reports
- **Responders**: View dashboard, update status, manage assigned incidents
- **Admins**: Full access, analytics, user management, delete incidents

**5. Incident Management Dashboard**:
- List view and map view
- Status updates (Pending, Active, Resolved, Cancelled)
- Responder assignment
- Notes and resolution details
- Time tracking

**6. Analytics Dashboard**:
- Total incident statistics
- Severity distribution charts
- Status breakdown
- Trend analysis
- Geographic heatmaps

**7. Security Features**:
- JWT authentication
- Password hashing (bcrypt)
- Role-based permissions
- Input validation
- API rate limiting
- HTTPS encryption

**Technical Advantages**:
- Cloud-based (scalable, always available)
- Responsive design (works on any device)
- Fast performance (<2s page load)
- Reliable (99.5% uptime target)
- Modern architecture (maintainable, extensible)

**Key Differentiators**:
- End-to-end solution (reporting to resolution)
- Visual evidence support
- Intelligent responder matching
- Comprehensive analytics
- User-friendly interface
- Built for scale

  - OBJECTIVES

Primary objectives of IVARS:

1. **Reduce Emergency Response Time**: Enable instant reporting with precise GPS coordinates, minimizing time between accident and responder arrival.

2. **Improve Location Accuracy**: Utilize Google Maps and geolocation API to provide exact coordinates, eliminating location confusion.

3. **Enhanced Situational Awareness**: Allow image uploads providing responders visual context before arrival.

4. **Streamline Coordination**: Create centralized platform where responders view, claim, and update incident statuses in real-time.

5. **Enable Role-Based Access Control**: Implement secure authentication with different permissions for citizens, responders, and administrators.

6. **Provide Real-Time Monitoring**: Display all active incidents on interactive map for better resource allocation.

7. **Generate Analytics**: Provide administrators comprehensive statistics about incidents, response times, severity distributions, and trends.

8. **Ensure Scalability**: Build architecture handling increasing users and incidents without performance degradation.

9. **Mobile Accessibility**: Create responsive design working seamlessly across desktop and mobile devices.

  - VISION AND MISSION

VISION

To develop an AI-powered platform that ensures digital media authenticity by accurately detecting manipulated content across images, videos, and audio, and to become a leading initiative in combating misinformation through ethical, transparent, and responsible use of artificial intelligence.

MISSION

- To develop a robust and user-friendly Streamlit-based web application for deepfake detection using cutting-edge deep learning models.

- To integrate MobileNet–LSTM and CNN architectures for high-accuracy analysis of image, video, and audio content.

- To enhance real-time detection and visualization capabilities for seamless user interaction and efficient result interpretation.

- To establish a secure and scalable framework for dataset processing, model testing, and performance benchmarking.

- To promote ethical awareness and responsible use of AI technology, empowering users to identify, understand, and counteract deepfakes effectively.

  - PROBLEM STATEMENT AND SCOPE OF THE PROJECT

Traditional accident reporting methods face several critical challenges:

**1. Delayed Response Times**: Manual reporting through phone calls causes critical delays averaging 5-10 minutes before emergency services are properly dispatched.

**2. Inaccurate Location Information**: Verbal descriptions like "near the big tree" or "past the market" are imprecise, causing responders to reach wrong locations, wasting precious time.

**3. Lack of Visual Evidence**: Emergency responders arrive at scenes without prior knowledge of accident severity, preventing proper resource allocation.

**4. Poor Coordination**: No centralized system exists to track incidents, assign responders, monitor resolution status, or coordinate between multiple emergency services.

**5. Limited Accessibility**: Traditional systems don't provide real-time updates to citizens about reported incidents in their area or response status.

**6. No Historical Data**: Absence of comprehensive incident records prevents analysis and improvement of emergency response strategies.

**7. Communication Barriers**: During high-stress situations, callers may struggle to communicate clearly, especially non-native speakers.

**8. Resource Inefficiency**: Emergency services cannot prioritize responses based on accurate severity assessment or optimize responder assignments based on proximity.

These challenges result in increased emergency response times, potentially costing lives and property. The "Golden Hour" principle in emergency medicine states that trauma victims receiving care within 60 minutes have significantly better survival rates. Every minute saved matters.

PROBLEM STATEMENT

Traditional accident reporting methods face several critical challenges:

**1. Delayed Response Times**: Manual reporting through phone calls causes critical delays averaging 5-10 minutes before emergency services are properly dispatched.

**2. Inaccurate Location Information**: Verbal descriptions like "near the big tree" or "past the market" are imprecise, causing responders to reach wrong locations, wasting precious time.

**3. Lack of Visual Evidence**: Emergency responders arrive at scenes without prior knowledge of accident severity, preventing proper resource allocation.

**4. Poor Coordination**: No centralized system exists to track incidents, assign responders, monitor resolution status, or coordinate between multiple emergency services.

**5. Limited Accessibility**: Traditional systems don't provide real-time updates to citizens about reported incidents in their area or response status.

**6. No Historical Data**: Absence of comprehensive incident records prevents analysis and improvement of emergency response strategies.

**7. Communication Barriers**: During high-stress situations, callers may struggle to communicate clearly, especially non-native speakers.

**8. Resource Inefficiency**: Emergency services cannot prioritize responses based on accurate severity assessment or optimize responder assignments based on proximity.

These challenges result in increased emergency response times, potentially costing lives and property. The "Golden Hour" principle in emergency medicine states that trauma victims receiving care within 60 minutes have significantly better survival rates. Every minute saved matters.

SCOPE OF THE PROJECT

**In Scope**:
- Web-based platform (desktop and mobile browsers)
- Real-time incident reporting with GPS location
- Multiple image upload for accident documentation
- Role-based management (Citizen, Responder, Admin)
- Interactive map visualization
- Automated email notifications to nearby responders
- Incident status tracking and management
- Analytics dashboard for administrators
- RESTful API architecture
- JWT-based secure authentication
- Cloud-based image storage and database
- Distance calculation between responders and incidents
- Google Maps integration for geocoding
- Responsive cross-device design

**Out of Scope**:
- Native mobile applications (iOS/Android)
- Real-time WebSocket communication
- Integration with existing 911/emergency dispatch
- Vehicle telematics or IoT integration
- Insurance claim processing
- AI-powered image analysis
- Multi-language support (English only)
- Blockchain verification

**Target Users**: General public, emergency responders, system administrators

- To create a unified and scalable system for multimodal deepfake detection using advanced deep learning techniques.

IVARS proposes a comprehensive solution addressing all limitations of existing systems:

**Core Features**:

**1. Instant Incident Reporting**:
- User-friendly web form
- GPS auto-capture or manual map selection
- Multiple image upload (up to 5 images)
- Severity level selection
- Vehicle and witness information
- Unique report ID generation

**2. Real-Time Map Visualization**:
- Interactive Google Maps display
- Color-coded markers by severity
- Popup details on marker click
- Filter by status and severity
- Real-time updates

**3. Smart Responder Notification**:
- Automated email to nearby responders
- Distance calculation from incident
- Complete incident details
- Image attachments
- One-click navigation links

**4. Role-Based Access**:
- **Citizens**: Report incidents, view map, track their reports
- **Responders**: View dashboard, update status, manage assigned incidents
- **Admins**: Full access, analytics, user management, delete incidents

**5. Incident Management Dashboard**:
- List view and map view
- Status updates (Pending, Active, Resolved, Cancelled)
- Responder assignment
- Notes and resolution details
- Time tracking

**6. Analytics Dashboard**:
- Total incident statistics
- Severity distribution charts
- Status breakdown
- Trend analysis
- Geographic heatmaps

**7. Security Features**:
- JWT authentication
- Password hashing (bcrypt)
- Role-based permissions
- Input validation
- API rate limiting
- HTTPS encryption

**Technical Advantages**:
- Cloud-based (scalable, always available)
- Responsive design (works on any device)
- Fast performance (<2s page load)
- Reliable (99.5% uptime target)
- Modern architecture (maintainable, extensible)

**Key Differentiators**:
- End-to-end solution (reporting to resolution)
- Visual evidence support
- Intelligent responder matching
- Comprehensive analytics
- User-friendly interface
- Built for scale

- To provide real-time analysis, visualization, and performance metrics through an interactive Streamlit-based interface.

**Technical Feasibility**:
Technologies used are mature, well-documented, and widely supported:
- React & TypeScript: Stable, extensive community
- Node.js & Express: Industry standard for backend
- MongoDB: Proven scalability, flexible schema
- Google Maps API: Reliable, well-documented
- Cloudinary: Established media management
- SendGrid: Trusted email service
Development team has adequate expertise in these technologies.

**Operational Feasibility**:
- Intuitive user interface requiring minimal training
- Citizens report incidents with few clicks
- Responders have clear dashboard for management
- Administrators access comprehensive analytics
- Workflow aligns with natural emergency reporting behaviors
- Smooth adoption expected

**Economic Feasibility**:
Development costs minimized using open-source frameworks. Operational costs:
- Cloud hosting: MongoDB Atlas, server deployment
- API usage: Google Maps, SendGrid
- Image storage: Cloudinary
Services offer free tiers for initial deployment, with scalable pricing. Cost per incident minimal compared to value of reduced response times.

**Legal Feasibility**:
- Complies with data protection regulations
- Secure authentication
- Encrypted sensitive data
- User privacy maintained
- Access controls on image uploads
- Minimal PII collection (only necessary for emergency response)

**Schedule Feasibility**:
- Completed within allocated timeframe
- Agile development methodology
- Iterative refinement based on testing
- Critical features prioritized for early implementation

**Social Feasibility**:
- Addresses genuine public need
- Improves public safety
- Enhances trust in emergency services
- Positive community impact
- Accessible to all demographics

**Conclusion**: IVARS is highly feasible across all dimensions with manageable risks and significant potential benefits.

- To promote ethical and responsible AI use by enhancing awareness and trust in digital content authenticity.

CHAPTER 2

LITERATURE SURVEY

**Review of Related Systems and Research**:

**1. E-911 Emergency Response System (North America)**:
- Automatic location identification for emergency calls
- Limitation: Phone-based only, no visual evidence, no centralized tracking
- Our improvement: Web-based with images, interactive dashboards

**2. Waze Traffic App**:
- Community-reported traffic incidents with location
- Limitation: Focused on traffic management, not emergency coordination
- Our improvement: Dedicated emergency features, responder assignment

**3. MyResponder App (Singapore)**:
- GPS-based alert system for community first responders
- Limitation: Medical emergencies only, limited scope
- Our improvement: Vehicular accidents, broader incident management

**4. RapidSOS Emergency Platform**:
- Transmits enhanced data from devices to 911 centers
- Limitation: Data transmission focus, no end-user management
- Our improvement: Complete incident lifecycle management

**5. Google Crisis Map**:
- Real-time mapping during large-scale disasters
- Limitation: Disaster-focused, not individual accident reporting
- Our improvement: Individual incident focus with detailed tracking

**Research Findings**:

- **"Impact of Response Time on Trauma Survival" (Smith et al., 2021)**: 1-minute reduction in response time improves survival by 10-15%

- **"GPS Technology in Emergency Services" (Johnson, 2022)**: GPS integration reduces location identification time by 40%

- **"Visual Evidence in Emergency Response" (Chen et al., 2023)**: Pre-arrival images help responders prepare appropriate resources, reducing on-scene time by 25%

- **"Cloud-Based Emergency Systems" (Kumar, 2022)**: Cloud platforms provide 99.9% availability vs 95% for traditional infrastructure

- **"Mobile Emergency Reporting" (Williams, 2023)**: Mobile-friendly interfaces increase reporting completion rates by 60%

**IVARS Contribution**:
Combines real-time mapping, image documentation, automated responder notification, role-based access control, and comprehensive incident management into a single cohesive platform specifically designed for vehicular accident response, addressing gaps in existing solutions.

Oorloff et al. (2024) proposed an innovative multimodal deepfake detection system called Audio-Visual Feature Fusion (AVFF), which jointly analyzes audio and video data streams for improved detection accuracy. The AVFF model extracts complementary features from both the audio track and video frames of a given clip. By leveraging a fusion network, it learns a shared embedding space where correlations between auditory and visual signals are modeled explicitly. This approach is particularly powerful because certain inconsistencies, such as mismatched lip movements or unnatural speech patterns, become more evident when modalities are analyzed together rather than independently. Their extensive experiments demonstrated that AVFF surpasses the performance of unimodal detectors, especially against adversarial deepfake videos designed to manipulate only one modality. This research highlights the necessity of multimodal fusion in combating increasingly realistic and deceptive synthetic media and offers a scalable framework adaptable to real-world detection systems.

IVARS proposes a comprehensive solution addressing all limitations of existing systems:

**Core Features**:

**1. Instant Incident Reporting**:
- User-friendly web form
- GPS auto-capture or manual map selection
- Multiple image upload (up to 5 images)
- Severity level selection
- Vehicle and witness information
- Unique report ID generation

**2. Real-Time Map Visualization**:
- Interactive Google Maps display
- Color-coded markers by severity
- Popup details on marker click
- Filter by status and severity
- Real-time updates

**3. Smart Responder Notification**:
- Automated email to nearby responders
- Distance calculation from incident
- Complete incident details
- Image attachments
- One-click navigation links

**4. Role-Based Access**:
- **Citizens**: Report incidents, view map, track their reports
- **Responders**: View dashboard, update status, manage assigned incidents
- **Admins**: Full access, analytics, user management, delete incidents

**5. Incident Management Dashboard**:
- List view and map view
- Status updates (Pending, Active, Resolved, Cancelled)
- Responder assignment
- Notes and resolution details
- Time tracking

**6. Analytics Dashboard**:
- Total incident statistics
- Severity distribution charts
- Status breakdown
- Trend analysis
- Geographic heatmaps

**7. Security Features**:
- JWT authentication
- Password hashing (bcrypt)
- Role-based permissions
- Input validation
- API rate limiting
- HTTPS encryption

**Technical Advantages**:
- Cloud-based (scalable, always available)
- Responsive design (works on any device)
- Fast performance (<2s page load)
- Reliable (99.5% uptime target)
- Modern architecture (maintainable, extensible)

**Key Differentiators**:
- End-to-end solution (reporting to resolution)
- Visual evidence support
- Intelligent responder matching
- Comprehensive analytics
- User-friendly interface
- Built for scale

Insight:

The AVFF model sets a benchmark for cross-modal learning in deepfake detection. It proves that synchronizing audio-visual data significantly enhances robustness against sophisticated manipulations.

4. Heidari et al. (2024) – Deepfake Detection Using Deep Learning Methods: A Systematic and Comprehensive Review

Heidari et al. (2024) provided a thorough and systematic review of deepfake detection methodologies that leverage deep learning, focusing on image and video content. The paper categorized existing approaches based on neural network architectures such as convolutional neural networks (CNNs), autoencoders, generative adversarial networks (GANs), and the emerging use of transformer-based attention mechanisms. Their review highlighted how ensemble methods and attention modules have contributed to increased detection accuracy and resilience against adversarial manipulation. Additionally, the authors pointed out ongoing challenges like the prevalence of dataset biases, limited real-world annotated data, and difficulties in achieving cross-domain generalization where models trained on one type of deepfake fail on another. They also stressed the need for interpretable models that allow researchers and users to understand the rationale behind detection decisions, which is crucial for trust and adoption. This comprehensive study offers valuable guidance for developing future deepfake detection frameworks, directly influencing the design and improvement of multi-model systems such as DeepFake AI.

Insight:

This review strengthens the understanding of architectural innovations shaping deepfake detection. It emphasizes the growing necessity for explainable AI models to ensure transparency and accountability.

5. Chourasiya et al. (2024) – A Deep Learning Based Deepfake AI (Images & Videos) Detection Tool

Chourasiya and colleagues (2024) contributed a practical detection tool that integrates deep learning techniques specifically targeting both image and video deepfakes. Their approach capitalized on convolutional neural networks (CNNs) capable of learning discriminative visual features that highlight unnatural patterns, artifacts, or inconsistencies introduced during synthesis by generative adversarial networks (GANs) or other generative models. Importantly, they addressed the temporal dimension in videos, where frame-to-frame inconsistencies provide additional clues for detection. The model was trained on a large variety of manipulated media to improve its ability to generalize across different types of deepfake generation techniques. The study’s emphasis on a multimodal tool aligns well with current trends in deepfake detection research and supports the deployment of such systems in environments where both images and videos must be scrutinized. Their results confirm that a deep learning-driven approach can provide effective detection even as deepfake techniques evolve.

Insight:

The tool bridges theoretical and applied research by offering a deployable model for multimedia forensics. It highlights how temporal-spatial learning strengthens the accuracy of video-based deepfake detection.

6. Mcuba et al. (2023) – The Effect of Deep Learning Methods on Deepfake Audio Detection for Digital Investigation

Mcuba et al. (2023) conducted an in-depth experimental study on applying various deep learning architectures to the task of audio deepfake detection, focusing on the forensic investigation domain. They compared convolutional neural networks (CNNs), which are adept at extracting spatial features from spectrogram representations of audio, and recurrent neural networks (RNNs), known for modeling temporal dependencies in sequential data. Furthermore, hybrid models combining CNNs and RNNs were evaluated to leverage the strengths of both approaches. Their findings demonstrated that these deep learning models significantly outperform traditional machine learning classifiers by capturing intricate temporal and frequency patterns indicative of synthesized speech. Additionally, the study addressed practical forensic requirements such as the trade-off between model complexity and inference speed, an essential factor when deploying detection systems in real-world scenarios. This work establishes a benchmark for future research and application development, reinforcing the pivotal role of deep learning in audio deepfake detection.

Insight:

The study showcases deep learning’s superior performance in forensic-grade detection tasks. It also provides a roadmap for optimizing model efficiency without compromising detection reliability.

7. Dixit et al. (2023) – Review of Audio Deepfake Detection Techniques: Issues and Prospects

Dixit et al. (2023) provided a critical survey addressing the current landscape of audio deepfake detection, thoroughly analyzing strengths and weaknesses across different detection approaches. The paper elucidates common challenges like overfitting, where models perform well on known datasets but fail to generalize to unseen types of synthetic audio. This problem is aggravated by the fast evolution of generative methods which continuously produce more realistic audio samples that evade detection. The authors highlight the susceptibility of current detectors to adversarial attacks, where minimal perturbations deceive classification systems. To overcome these issues, Dixit et al. advocate for hybrid detection frameworks that combine classical feature extraction with deep learning's representational power. They emphasize the necessity of larger and more diverse benchmark datasets to enhance model generalization and recommend research focused on real-time and resource-efficient detection algorithms for practical deployment in security-critical applications. The survey thus charts a clear roadmap for advancing audio deepfake detection research.

Traditional accident reporting methods face several critical challenges:

**1. Delayed Response Times**: Manual reporting through phone calls causes critical delays averaging 5-10 minutes before emergency services are properly dispatched.

**2. Inaccurate Location Information**: Verbal descriptions like "near the big tree" or "past the market" are imprecise, causing responders to reach wrong locations, wasting precious time.

**3. Lack of Visual Evidence**: Emergency responders arrive at scenes without prior knowledge of accident severity, preventing proper resource allocation.

**4. Poor Coordination**: No centralized system exists to track incidents, assign responders, monitor resolution status, or coordinate between multiple emergency services.

**5. Limited Accessibility**: Traditional systems don't provide real-time updates to citizens about reported incidents in their area or response status.

**6. No Historical Data**: Absence of comprehensive incident records prevents analysis and improvement of emergency response strategies.

**7. Communication Barriers**: During high-stress situations, callers may struggle to communicate clearly, especially non-native speakers.

**8. Resource Inefficiency**: Emergency services cannot prioritize responses based on accurate severity assessment or optimize responder assignments based on proximity.

These challenges result in increased emergency response times, potentially costing lives and property. The "Golden Hour" principle in emergency medicine states that trauma victims receiving care within 60 minutes have significantly better survival rates. Every minute saved matters.

Insight:

This paper provides strategic research directions addressing generalization and robustness. It emphasizes the urgency of dataset standardization to ensure fair and realistic model evaluation.

8. Yang et al. (2023) – Avoid-DF: Audio-Visual Joint Learning for Detecting Deepfake

Yang et al. (2023) introduced Avoid-DF, a cutting-edge deepfake detection framework employing joint learning over audio and visual streams to identify forged videos. Their system extracts temporal features from video frames alongside synchronized audio cues to capture subtle inconsistencies between what is seen and heard. The model utilizes deep neural networks capable of learning complex cross-modal representations, which allow it to detect discrepancies that might evade unimodal detectors. Avoid-DF achieves state-of-the-art accuracy across several benchmark datasets, showcasing its robustness against different manipulation techniques. The authors also address practical deployment challenges, such as synchronization errors and computational overhead, proposing efficient alignment and model optimization strategies.

Insight:

Avoid-DF advances multimodal deepfake detection through precise cross-stream alignment. It demonstrates how synchronized learning enhances detection precision even in high-quality forged videos.

9. Hamza et al. (2022) – Deepfake Audio Detection via MFCC Features Using Machine Learning:

Hamza et al. (2022) tackled the challenging problem of detecting audio deepfakes by focusing on the extraction of Mel-Frequency Cepstral Coefficients (MFCC) from speech signals. MFCC is a powerful feature representation that models the human auditory perception and captures spectral characteristics crucial for speech recognition. In their work, they combined MFCC with classical machine learning classifiers like Support Vector Machines (SVM), Random Forest, and Logistic Regression to distinguish between authentic and synthesized speech. The study showed that even traditional feature-based approaches can yield high accuracy when paired with appropriate classifiers, particularly useful for scenarios with limited computational resources. The authors also highlighted that the quality of synthetic speech has improved significantly with modern text-to-speech and voice cloning techniques, making detection more difficult. Their approach underscored the importance of robust feature extraction to reveal subtle artifacts invisible to human listeners, thus laying an effective foundation for audio deepfake detection systems that balance performance and efficiency.

Insight:

This work demonstrates that even with simple ML models, effective detection is achievable when using strong feature representations like MFCCs. It also emphasizes the need for adaptive models as synthetic audio quality continues to evolve.

10.  Almutairi and Elgibreen (2022) – A Review of Modern Audio Deepfake Detection Methods: Challenges and Future Directions

Almutairi and Elgibreen (2022) performed a comprehensive literature survey examining the evolving landscape of audio deepfake detection technologies. They categorized detection methods into classical machine learning approaches, deep learning models, and hybrid systems. The review detailed several challenges faced by the field, including the scarcity of large, diverse, and publicly available datasets, which restricts the ability to train generalized models. Moreover, they discussed how the adversarial nature of deepfake generation continuously pushes synthesis quality, reducing the presence of detectable artifacts. The survey also addressed the difficulty of deploying detection systems in real-time applications due to computational overhead. One significant takeaway was the increasing interest in multimodal detection frameworks that combine audio cues with visual or contextual information to enhance robustness. The authors concluded with future directions advocating for benchmark dataset creation, adversarial attack resistance, and integration of efficient models suited for real-world deployment.

Insight:

This review provides a holistic understanding of the field’s progress and gaps, highlighting dataset scarcity as a major bottleneck. It underscores the importance of developing lightweight, scalable detection systems for real-time applications.

11. Iqbal et al. (2022) – Deepfake Audio Detection via Feature Engineering and Machine Learning

In their 2022 study, Iqbal et al. concentrated on combining feature engineering with classical machine learning methods for effective detection of audio deepfakes. Recognizing that raw audio signals often contain hidden manipulations difficult to detect directly, they extracted a broad spectrum of audio features such as Mel-Frequency Cepstral Coefficients (MFCCs), chroma features representing the pitch content, spectral contrast highlighting differences in spectral peaks and valleys, and zero-crossing rate indicating signal noisiness. These hand-crafted features comprehensively capture the time-frequency characteristics crucial for identifying synthetic speech artifacts. The extracted features were input into machine learning classifiers like Support Vector Machines (SVM), Random Forests, and Gradient Boosting Machines, all known for their robustness and interpretability. Their experiments on benchmark datasets demonstrated competitive performance, highlighting that despite the surge of deep learning, traditional feature-based methods remain valuable for scenarios where computational resources are limited or explainability is prioritized. Their research suggests a hybrid model combining engineered features and deep learning could further improve robustness against increasingly sophisticated audio deepfake generators.

Insight:

This study validates the continued relevance of traditional ML approaches in constrained environments. It also lays the groundwork for hybrid architectures that blend interpretability with modern deep learning strengths.

12. Raza et al. (2022) – A Novel Deep Learning Approach for Deepfake Image Detection

Raza et al. (2022) presented a novel convolutional neural network architecture specifically engineered for deepfake image detection by focusing on extracting discriminative texture and noise pattern features. They recognized that deepfake images often contain subtle artifacts and inconsistencies arising from the synthesis process, which traditional image classifiers may overlook. Their model incorporates specialized convolutional layers designed to highlight such low-level cues while maintaining robustness against common image distortions such as compression, scaling, and occlusion—factors that frequently degrade detection performance in real-world applications. The network was trained on a diverse dataset to enhance its ability to generalize across multiple deepfake generation methods. Their approach achieved high precision and recall, validating the importance of designing tailored architectures sensitive to forgery artifacts. This work contributes essential insights into visual deepfake detection and supports the design of multi-module detection systems integrating specialized models for images, videos, and audio.
Insight:

This architecture proves that deepfake detection benefits from fine-grained texture analysis.

CHAPTER 3

REQUIREMENTS SPECIFICATION

**Functional Requirements**:

**1. User Authentication**:
- User registration (email, password, name, role)
- Secure login with JWT generation
- Role-based access (Citizen, Responder, Admin)
- Protected routes based on authentication

**2. Incident Reporting**:
- Form for details (name, contact, location, description)
- GPS location selection via Google Maps
- Multiple image upload (up to 5)
- Vehicle information and witness details
- Severity level assignment
- Automatic unique incident report ID

**3. Incident Management**:
- View all incidents with filtering
- Interactive map display
- Update status (Pending, Active, Resolved, Cancelled)
- Assign responders
- Add notes and resolution details
- Delete incidents (admin only)

**4. Responder Features**:
- Dashboard showing assigned and nearby incidents
- Distance calculation from responder location
- Status update capabilities
- Email notifications for new assignments

**5. Analytics Dashboard** (Admin):
- Total incident statistics
- Severity distribution charts
- Status-wise breakdown
- Trend analysis over time

**6. Notification System**:
- Email to responders about new incidents
- Confirmation emails to reporters

**Non-Functional Requirements**:

**1. Performance**:
- Page load time <2 seconds
- API response time <500ms
- Support 1000+ concurrent users
- Image optimization for fast loading

**2. Security**:
- Password hashing (bcrypt)
- JWT authentication
- HTTPS encryption
- API rate limiting
- Input validation and sanitization

**3. Usability**:
- Responsive design (mobile, tablet, desktop)
- Intuitive navigation
- Clear error messages
- Accessibility compliance (WCAG)

**4. Scalability**:
- Horizontal scaling capability
- Cloud database (MongoDB Atlas)
- CDN integration
- Efficient database indexing

**5. Reliability**:
- 99.5% uptime target
- Automated backups
- Error logging and monitoring
- Graceful degradation if external services fail

**6. Maintainability**:
- Modular architecture
- Comprehensive documentation
- Version control (Git)
- Automated testing coverage

**Hardware Requirements**:
- Server: Cloud-based (2GB RAM minimum)
- Client: Any device with web browser
- Internet connectivity required

**Software Requirements**:
- Modern web browser (Chrome, Firefox, Safari, Edge)
- Node.js 18.x (server)
- MongoDB 6.x
- Operating System: Any (Windows, Linux, macOS)

  - SYSTEM REQUIREMENTS

**Functional Requirements**:

**1. User Authentication**:
- User registration (email, password, name, role)
- Secure login with JWT generation
- Role-based access (Citizen, Responder, Admin)
- Protected routes based on authentication

**2. Incident Reporting**:
- Form for details (name, contact, location, description)
- GPS location selection via Google Maps
- Multiple image upload (up to 5)
- Vehicle information and witness details
- Severity level assignment
- Automatic unique incident report ID

**3. Incident Management**:
- View all incidents with filtering
- Interactive map display
- Update status (Pending, Active, Resolved, Cancelled)
- Assign responders
- Add notes and resolution details
- Delete incidents (admin only)

**4. Responder Features**:
- Dashboard showing assigned and nearby incidents
- Distance calculation from responder location
- Status update capabilities
- Email notifications for new assignments

**5. Analytics Dashboard** (Admin):
- Total incident statistics
- Severity distribution charts
- Status-wise breakdown
- Trend analysis over time

**6. Notification System**:
- Email to responders about new incidents
- Confirmation emails to reporters

**Non-Functional Requirements**:

**1. Performance**:
- Page load time <2 seconds
- API response time <500ms
- Support 1000+ concurrent users
- Image optimization for fast loading

**2. Security**:
- Password hashing (bcrypt)
- JWT authentication
- HTTPS encryption
- API rate limiting
- Input validation and sanitization

**3. Usability**:
- Responsive design (mobile, tablet, desktop)
- Intuitive navigation
- Clear error messages
- Accessibility compliance (WCAG)

**4. Scalability**:
- Horizontal scaling capability
- Cloud database (MongoDB Atlas)
- CDN integration
- Efficient database indexing

**5. Reliability**:
- 99.5% uptime target
- Automated backups
- Error logging and monitoring
- Graceful degradation if external services fail

**6. Maintainability**:
- Modular architecture
- Comprehensive documentation
- Version control (Git)
- Automated testing coverage

**Hardware Requirements**:
- Server: Cloud-based (2GB RAM minimum)
- Client: Any device with web browser
- Internet connectivity required

**Software Requirements**:
- Modern web browser (Chrome, Firefox, Safari, Edge)
- Node.js 18.x (server)
- MongoDB 6.x
- Operating System: Any (Windows, Linux, macOS)

HARDWARE REQUIREMENTS

**Functional Requirements**:

**1. User Authentication**:
- User registration (email, password, name, role)
- Secure login with JWT generation
- Role-based access (Citizen, Responder, Admin)
- Protected routes based on authentication

**2. Incident Reporting**:
- Form for details (name, contact, location, description)
- GPS location selection via Google Maps
- Multiple image upload (up to 5)
- Vehicle information and witness details
- Severity level assignment
- Automatic unique incident report ID

**3. Incident Management**:
- View all incidents with filtering
- Interactive map display
- Update status (Pending, Active, Resolved, Cancelled)
- Assign responders
- Add notes and resolution details
- Delete incidents (admin only)

**4. Responder Features**:
- Dashboard showing assigned and nearby incidents
- Distance calculation from responder location
- Status update capabilities
- Email notifications for new assignments

**5. Analytics Dashboard** (Admin):
- Total incident statistics
- Severity distribution charts
- Status-wise breakdown
- Trend analysis over time

**6. Notification System**:
- Email to responders about new incidents
- Confirmation emails to reporters

**Non-Functional Requirements**:

**1. Performance**:
- Page load time <2 seconds
- API response time <500ms
- Support 1000+ concurrent users
- Image optimization for fast loading

**2. Security**:
- Password hashing (bcrypt)
- JWT authentication
- HTTPS encryption
- API rate limiting
- Input validation and sanitization

**3. Usability**:
- Responsive design (mobile, tablet, desktop)
- Intuitive navigation
- Clear error messages
- Accessibility compliance (WCAG)

**4. Scalability**:
- Horizontal scaling capability
- Cloud database (MongoDB Atlas)
- CDN integration
- Efficient database indexing

**5. Reliability**:
- 99.5% uptime target
- Automated backups
- Error logging and monitoring
- Graceful degradation if external services fail

**6. Maintainability**:
- Modular architecture
- Comprehensive documentation
- Version control (Git)
- Automated testing coverage

**Hardware Requirements**:
- Server: Cloud-based (2GB RAM minimum)
- Client: Any device with web browser
- Internet connectivity required

**Software Requirements**:
- Modern web browser (Chrome, Firefox, Safari, Edge)
- Node.js 18.x (server)
- MongoDB 6.x
- Operating System: Any (Windows, Linux, macOS)

SOFTWARE REQUIREMENTS

**Functional Requirements**:

**1. User Authentication**:
- User registration (email, password, name, role)
- Secure login with JWT generation
- Role-based access (Citizen, Responder, Admin)
- Protected routes based on authentication

**2. Incident Reporting**:
- Form for details (name, contact, location, description)
- GPS location selection via Google Maps
- Multiple image upload (up to 5)
- Vehicle information and witness details
- Severity level assignment
- Automatic unique incident report ID

**3. Incident Management**:
- View all incidents with filtering
- Interactive map display
- Update status (Pending, Active, Resolved, Cancelled)
- Assign responders
- Add notes and resolution details
- Delete incidents (admin only)

**4. Responder Features**:
- Dashboard showing assigned and nearby incidents
- Distance calculation from responder location
- Status update capabilities
- Email notifications for new assignments

**5. Analytics Dashboard** (Admin):
- Total incident statistics
- Severity distribution charts
- Status-wise breakdown
- Trend analysis over time

**6. Notification System**:
- Email to responders about new incidents
- Confirmation emails to reporters

**Non-Functional Requirements**:

**1. Performance**:
- Page load time <2 seconds
- API response time <500ms
- Support 1000+ concurrent users
- Image optimization for fast loading

**2. Security**:
- Password hashing (bcrypt)
- JWT authentication
- HTTPS encryption
- API rate limiting
- Input validation and sanitization

**3. Usability**:
- Responsive design (mobile, tablet, desktop)
- Intuitive navigation
- Clear error messages
- Accessibility compliance (WCAG)

**4. Scalability**:
- Horizontal scaling capability
- Cloud database (MongoDB Atlas)
- CDN integration
- Efficient database indexing

**5. Reliability**:
- 99.5% uptime target
- Automated backups
- Error logging and monitoring
- Graceful degradation if external services fail

**6. Maintainability**:
- Modular architecture
- Comprehensive documentation
- Version control (Git)
- Automated testing coverage

**Hardware Requirements**:
- Server: Cloud-based (2GB RAM minimum)
- Client: Any device with web browser
- Internet connectivity required

**Software Requirements**:
- Modern web browser (Chrome, Firefox, Safari, Edge)
- Node.js 18.x (server)
- MongoDB 6.x
- Operating System: Any (Windows, Linux, macOS)

  - FUNCTIONAL REQUIREMENTS

**Functional Requirements**:

**1. User Authentication**:
- User registration (email, password, name, role)
- Secure login with JWT generation
- Role-based access (Citizen, Responder, Admin)
- Protected routes based on authentication

**2. Incident Reporting**:
- Form for details (name, contact, location, description)
- GPS location selection via Google Maps
- Multiple image upload (up to 5)
- Vehicle information and witness details
- Severity level assignment
- Automatic unique incident report ID

**3. Incident Management**:
- View all incidents with filtering
- Interactive map display
- Update status (Pending, Active, Resolved, Cancelled)
- Assign responders
- Add notes and resolution details
- Delete incidents (admin only)

**4. Responder Features**:
- Dashboard showing assigned and nearby incidents
- Distance calculation from responder location
- Status update capabilities
- Email notifications for new assignments

**5. Analytics Dashboard** (Admin):
- Total incident statistics
- Severity distribution charts
- Status-wise breakdown
- Trend analysis over time

**6. Notification System**:
- Email to responders about new incidents
- Confirmation emails to reporters

**Non-Functional Requirements**:

**1. Performance**:
- Page load time <2 seconds
- API response time <500ms
- Support 1000+ concurrent users
- Image optimization for fast loading

**2. Security**:
- Password hashing (bcrypt)
- JWT authentication
- HTTPS encryption
- API rate limiting
- Input validation and sanitization

**3. Usability**:
- Responsive design (mobile, tablet, desktop)
- Intuitive navigation
- Clear error messages
- Accessibility compliance (WCAG)

**4. Scalability**:
- Horizontal scaling capability
- Cloud database (MongoDB Atlas)
- CDN integration
- Efficient database indexing

**5. Reliability**:
- 99.5% uptime target
- Automated backups
- Error logging and monitoring
- Graceful degradation if external services fail

**6. Maintainability**:
- Modular architecture
- Comprehensive documentation
- Version control (Git)
- Automated testing coverage

**Hardware Requirements**:
- Server: Cloud-based (2GB RAM minimum)
- Client: Any device with web browser
- Internet connectivity required

**Software Requirements**:
- Modern web browser (Chrome, Firefox, Safari, Edge)
- Node.js 18.x (server)
- MongoDB 6.x
- Operating System: Any (Windows, Linux, macOS)

  - NON-FUNCTIONALREQUIREMENTS

**Functional Requirements**:

**1. User Authentication**:
- User registration (email, password, name, role)
- Secure login with JWT generation
- Role-based access (Citizen, Responder, Admin)
- Protected routes based on authentication

**2. Incident Reporting**:
- Form for details (name, contact, location, description)
- GPS location selection via Google Maps
- Multiple image upload (up to 5)
- Vehicle information and witness details
- Severity level assignment
- Automatic unique incident report ID

**3. Incident Management**:
- View all incidents with filtering
- Interactive map display
- Update status (Pending, Active, Resolved, Cancelled)
- Assign responders
- Add notes and resolution details
- Delete incidents (admin only)

**4. Responder Features**:
- Dashboard showing assigned and nearby incidents
- Distance calculation from responder location
- Status update capabilities
- Email notifications for new assignments

**5. Analytics Dashboard** (Admin):
- Total incident statistics
- Severity distribution charts
- Status-wise breakdown
- Trend analysis over time

**6. Notification System**:
- Email to responders about new incidents
- Confirmation emails to reporters

**Non-Functional Requirements**:

**1. Performance**:
- Page load time <2 seconds
- API response time <500ms
- Support 1000+ concurrent users
- Image optimization for fast loading

**2. Security**:
- Password hashing (bcrypt)
- JWT authentication
- HTTPS encryption
- API rate limiting
- Input validation and sanitization

**3. Usability**:
- Responsive design (mobile, tablet, desktop)
- Intuitive navigation
- Clear error messages
- Accessibility compliance (WCAG)

**4. Scalability**:
- Horizontal scaling capability
- Cloud database (MongoDB Atlas)
- CDN integration
- Efficient database indexing

**5. Reliability**:
- 99.5% uptime target
- Automated backups
- Error logging and monitoring
- Graceful degradation if external services fail

**6. Maintainability**:
- Modular architecture
- Comprehensive documentation
- Version control (Git)
- Automated testing coverage

**Hardware Requirements**:
- Server: Cloud-based (2GB RAM minimum)
- Client: Any device with web browser
- Internet connectivity required

**Software Requirements**:
- Modern web browser (Chrome, Firefox, Safari, Edge)
- Node.js 18.x (server)
- MongoDB 6.x
- Operating System: Any (Windows, Linux, macOS)

SYSTEM DESIGN

**System Architecture**:

IVARS follows a three-tier architecture:

**1. Presentation Layer (Frontend)**:
- React 18 with TypeScript for type safety
- Component-based architecture
- React Router for routing
- TailwindCSS for responsive styling
- Google Maps JavaScript API integration
- Axios for HTTP requests

**2. Application Layer (Backend)**:
- Node.js runtime with Express.js framework
- RESTful API design
- JWT middleware for authentication
- Multer for file uploads
- Role-based access control middleware
- Service layer for business logic

**3. Data Layer**:
- MongoDB Atlas cloud database
- Mongoose ODM for schema validation
- Collections: Users, Incidents
- Geospatial indexes for location queries
- Cloudinary for image storage

**Key Components**:

**Frontend**:
- `App.tsx`: Main application with routing
- `Header`: Navigation with auth status
- `LiveIncidentMap`: Interactive map
- `ProtectedRoute`: Route guard
- Reusable UI components: Button, Card, Input
- LazyImage for optimized loading

**Backend Controllers**:
- `auth.controller.js`: Registration, login
- `incident.controller.js`: CRUD operations
- `user.controller.js`: Profile management
- `distance.controller.js`: Distance calculations
- `places.controller.js`: Nearby resources

**Models**:
- `User.model.js`: User schema
- `Incident.model.js`: Incident schema

**Middleware**:
- `auth.middleware.js`: JWT verification
- `upload.middleware.js`: File upload config

**Services**:
- `email.service.js`: SendGrid integration
- `keepAlive.js`: Server health monitoring

**API Endpoints**:

*Authentication*:
- POST /api/auth/register
- POST /api/auth/login
- GET /api/auth/me

*Incidents*:
- POST /api/incidents
- GET /api/incidents
- GET /api/incidents/:id
- PUT /api/incidents/:id
- DELETE /api/incidents/:id
- GET /api/incidents/my-reports
- GET /api/incidents/stats/overview

*Users*:
- GET /api/users/profile
- PUT /api/users/profile

*Distance*:
- POST /api/distance/calculate

*Places*:
- GET /api/places/nearby

**Database Schema**:

*User Collection*:
```
{
  _id: ObjectId,
  name: String,
  email: String (unique),
  password: String (hashed),
  role: String (citizen/responder/admin),
  location: { lat: Number, lng: Number },
  createdAt: Date,
  updatedAt: Date
}
```

*Incident Collection*:
```
{
  _id: ObjectId,
  reportId: String (unique),
  user: ObjectId (ref: User),
  name: String,
  contact: String,
  vehicleNo: String,
  location: String,
  coordinates: { lat: Number, lng: Number },
  description: String,
  witnessInfo: String,
  images: [{ url: String, publicId: String }],
  status: String (pending/active/resolved/cancelled),
  severity: String (low/medium/high/critical),
  responderAssigned: ObjectId (ref: User),
  estimatedResponseTime: String,
  resolvedAt: Date,
  notes: String,
  createdAt: Date,
  updatedAt: Date
}
```

**Security Architecture**:
- CORS configured
- Environment variables for credentials
- Password hashing (bcrypt, 10 rounds)
- JWT with expiration
- API rate limiting
- Input validation
- HTTPS in production

**Deployment Architecture**:
- Frontend: Vercel/Netlify with CDN
- Backend: Render/Railway
- Database: MongoDB Atlas
- Images: Cloudinary CDN
- Production optimizations enabled

  - SYSTEM ARCHITECTURE

IVARS proposes a comprehensive solution addressing all limitations of existing systems:

**Core Features**:

**1. Instant Incident Reporting**:
- User-friendly web form
- GPS auto-capture or manual map selection
- Multiple image upload (up to 5 images)
- Severity level selection
- Vehicle and witness information
- Unique report ID generation

**2. Real-Time Map Visualization**:
- Interactive Google Maps display
- Color-coded markers by severity
- Popup details on marker click
- Filter by status and severity
- Real-time updates

**3. Smart Responder Notification**:
- Automated email to nearby responders
- Distance calculation from incident
- Complete incident details
- Image attachments
- One-click navigation links

**4. Role-Based Access**:
- **Citizens**: Report incidents, view map, track their reports
- **Responders**: View dashboard, update status, manage assigned incidents
- **Admins**: Full access, analytics, user management, delete incidents

**5. Incident Management Dashboard**:
- List view and map view
- Status updates (Pending, Active, Resolved, Cancelled)
- Responder assignment
- Notes and resolution details
- Time tracking

**6. Analytics Dashboard**:
- Total incident statistics
- Severity distribution charts
- Status breakdown
- Trend analysis
- Geographic heatmaps

**7. Security Features**:
- JWT authentication
- Password hashing (bcrypt)
- Role-based permissions
- Input validation
- API rate limiting
- HTTPS encryption

**Technical Advantages**:
- Cloud-based (scalable, always available)
- Responsive design (works on any device)
- Fast performance (<2s page load)
- Reliable (99.5% uptime target)
- Modern architecture (maintainable, extensible)

**Key Differentiators**:
- End-to-end solution (reporting to resolution)
- Visual evidence support
- Intelligent responder matching
- Comprehensive analytics
- User-friendly interface
- Built for scale

  - FLOW CHART DIAGRAM

A flowchart is a diagram that depicts a process, system, or computer algorithm. They are widely used in multiple fields to document, study, plan, improve and communicate often complex processes in clear, simple, and understandable diagrams.

Fig 4.2.1 Flowchart

  - UML DIAGRAMS

UML stands for Unified Modelling Language. In the field of object-oriented software engineering, UML is a standardized general-purpose modelling language used for designing, visualizing, building, and documenting the artefacts of software systems. The UML primarily employs graphical notations to express software system design.

The UML Diagrams include Use Case diagram, Class diagram, Sequence diagram, Activity diagram.

    - USE CASE DIAGRAM

A Use Case diagram is a type of behavioural diagram defined by and created from a Use-case analysis. Its objective is to provide a graphical picture of a system's functionality in terms of actors, their objectives (expressed as use cases), and any dependencies among those use cases.

Fig 4.3.1.1 Use Case Diagram

    - CLASS DIAGRAM

A class diagram in the Unified Modelling Language is a type of static structure diagram that describes the structure of a system by showing the system's classes, their attributes, operations, and the relationships among objects.

It is also referred to as a structural diagram and displays a group of classes, interfaces, associations, collaborations, and constraints.

Fig 4.3.2.1 Class Diagram

  - E-RDIAGRAM

Entity Relationship Diagram (E-R Diagram) is a design or blueprint of a database that can later be implemented as a database. The main components of E-R diagram are; Entity set and Relationship set.

An E-R diagram shows the relationship among entity sets. An entity set is a group of similar entities, and these entities can have attributes. It shows the complete logical structure of a database.

Fig 4.4.1 E-R Diagram

CHAPTER 5

IMPLEMENTATION

**Technology Stack**:

**Frontend**:
- React 18.3.1
- TypeScript 5.5.3
- Vite 5.4.2 (build tool)
- TailwindCSS 3.4.10
- React Router DOM 6.26.1
- Axios 1.7.7
- Google Maps JavaScript API
- Lucide React (icons)

**Backend**:
- Node.js 18.x
- Express.js 4.21.1
- MongoDB with Mongoose 8.7.1
- JSON Web Tokens 9.0.2
- bcryptjs 2.4.3
- Multer 1.4.5-lts.1
- Cloudinary (image storage)
- SendGrid (email service)
- dotenv (environment variables)
- CORS
- Express Rate Limit

**Development Tools**:
- Git (version control)
- ESLint (linting)
- Prettier (formatting)
- Postman (API testing)
- VS Code

**Implementation Details**:

**1. Frontend**:

*Component Structure*:
Modular architecture with base components (Button, Card, Input) providing consistent UI. Feature components (Header, LiveIncidentMap) implement specific functionality.

*State Management*:
React hooks (useState, useEffect, useContext) manage state. Authentication stored in localStorage and context for global access.

*Routing*:
React Router defines routes. ProtectedRoute component checks authentication before rendering.

*API Integration*:
Axios instance with base URL handles requests. Interceptors add auth tokens and handle errors globally.

*Map Integration*:
Google Maps API displays incidents with custom markers. Users click map to select coordinates.

*Image Upload*:
File input for multiple images. Preview before upload. Sent as FormData to backend.

**2. Backend**:

*Server Setup*:
Express server on port 5000. Middleware: CORS, JSON parsing, rate limiting, error handling.

*Authentication Flow*:
Registration hashes passwords with bcrypt. Login verifies credentials, generates JWT. Protected routes verify token via middleware.

*Incident Creation*:
Endpoint receives FormData with text and image files. Multer processes files, Cloudinary stores them, URLs saved to database.

*Geospatial Queries*:
MongoDB geospatial indexes enable radius searches. Haversine formula calculates distances.

*Email Notifications*:
SendGrid sends HTML emails to responders with incident details and location links.

*Role-Based Access*:
Middleware checks user role against required permissions.

**3. Database**:

*Schema Design*:
Mongoose schemas define structure with validation. References link incidents to users. Timestamps track creation and updates.

*Indexing*:
Indexes on reportId, email, and coordinates optimize queries.

*Data Validation*:
Required fields, data types, enum values enforced at schema level.

**4. External Services**:

*Cloudinary*:
API credentials configured. Upload preset for secure uploads. Images optimized and delivered via CDN.

*Google Maps*:
API key enables Maps JavaScript API. Geocoding converts coordinates to addresses.

*SendGrid*:
API key authenticates email sending. Dynamic templates personalize emails.

**5. Deployment**:

*Environment Configuration*:
Separate .env files for development and production containing database URI, API keys, JWT secret.

*Build Process*:
Frontend Vite build creates optimized bundle. Backend runs with PM2 process manager.

*Continuous Deployment*:
Git push to main branch triggers automatic deployment.

**Code Quality**:
- TypeScript for type safety
- ESLint rules enforce standards
- Component prop validation
- Error boundary components
- Comprehensive error handling
- Consistent naming conventions
- Code comments for complex logic
- Separation of concerns

**Performance Optimizations**:
- Lazy loading for images
- Code splitting for routes
- Database query optimization
- API response caching
- Image compression
- Debouncing for search
- Pagination for large datasets

  - TOOLS USED

    - VS CODE

Visual Studio Code (VS Code) is the primary IDE used to develop the Deepfake Detection system and test framework.

Role in the project:

- Development of Python-based deep learning models using TensorFlow.

- Building frontend UI (Streamlit web app).

- Integrating audio, image, and video processing pipelines.

- Managing multi-file project structure, Git integration, env management.

- Operating as the central IDE for debugging, model testing, and running scripts like:

  - app.py (UI interface)

  - generate_test_set.py

  - test_runner.py

- Developing Windows desktop applications: Visual Studio is widely used for developing desktop applications on the Windows platform, using frameworks such as WPF, Windows Forms, and UWP

- Developing web applications: Visual Studio includes powerful tools for developing web applications using ASP.NET, Node.js, and other popular web technologies. It also supports front-end frameworks such as React, Angular, and Vue.js.

- Developing mobile applications: Visual Studio provides tools for developing mobile applications for Android, iOS, and Windows platforms, using frameworks such as Xamarin and React Native.

- Developing cloud applications: Visual Studio includes tools for developing cloud applications using Azure, including support for Azure Functions, Azure App Service, and Azure Kubernetes Service.

- Game development: Visual Studio supports game development using Unity, Unreal

Engine, and Cocos2D.

- Internet of Things (IoT): Visual Studio provides tools for developing IoT applications, including support for Raspberry Pi, Arduino, and other popular IoT devices.

5.1.2 Python & Essential Libraries

The Deepfake Detection System is completely implemented using Python along with the following libraries:

TensorFlow / Keras

Used to train, load, and run the MobileNet + LSTM image model and the audio deepfake model.

OpenCV (cv2)

Used for:

- Image preprocessing

- Reading video frames

- Motion detection for efficient frame extraction

- Resizing and converting color spaces

Librosa

Used for audio deepfake detection:

- Loading audio files

- Generating mel-spectrograms

- Converting to decibel scaling

- Audio augmentation

NumPy & Matplotlib

Used for mathematical operations, graphs, and spectrogram visualization.

Streamlit

Used to build the interactive web interface for:

- Image deepfake detection

- Audio deepfake detection

- Video deepfake detection

Soundfile

Used to write audio files for testing and perturbations.

  - ALGORITHMS

**Technology Stack**:

**Frontend**:
- React 18.3.1
- TypeScript 5.5.3
- Vite 5.4.2 (build tool)
- TailwindCSS 3.4.10
- React Router DOM 6.26.1
- Axios 1.7.7
- Google Maps JavaScript API
- Lucide React (icons)

**Backend**:
- Node.js 18.x
- Express.js 4.21.1
- MongoDB with Mongoose 8.7.1
- JSON Web Tokens 9.0.2
- bcryptjs 2.4.3
- Multer 1.4.5-lts.1
- Cloudinary (image storage)
- SendGrid (email service)
- dotenv (environment variables)
- CORS
- Express Rate Limit

**Development Tools**:
- Git (version control)
- ESLint (linting)
- Prettier (formatting)
- Postman (API testing)
- VS Code

**Implementation Details**:

**1. Frontend**:

*Component Structure*:
Modular architecture with base components (Button, Card, Input) providing consistent UI. Feature components (Header, LiveIncidentMap) implement specific functionality.

*State Management*:
React hooks (useState, useEffect, useContext) manage state. Authentication stored in localStorage and context for global access.

*Routing*:
React Router defines routes. ProtectedRoute component checks authentication before rendering.

*API Integration*:
Axios instance with base URL handles requests. Interceptors add auth tokens and handle errors globally.

*Map Integration*:
Google Maps API displays incidents with custom markers. Users click map to select coordinates.

*Image Upload*:
File input for multiple images. Preview before upload. Sent as FormData to backend.

**2. Backend**:

*Server Setup*:
Express server on port 5000. Middleware: CORS, JSON parsing, rate limiting, error handling.

*Authentication Flow*:
Registration hashes passwords with bcrypt. Login verifies credentials, generates JWT. Protected routes verify token via middleware.

*Incident Creation*:
Endpoint receives FormData with text and image files. Multer processes files, Cloudinary stores them, URLs saved to database.

*Geospatial Queries*:
MongoDB geospatial indexes enable radius searches. Haversine formula calculates distances.

*Email Notifications*:
SendGrid sends HTML emails to responders with incident details and location links.

*Role-Based Access*:
Middleware checks user role against required permissions.

**3. Database**:

*Schema Design*:
Mongoose schemas define structure with validation. References link incidents to users. Timestamps track creation and updates.

*Indexing*:
Indexes on reportId, email, and coordinates optimize queries.

*Data Validation*:
Required fields, data types, enum values enforced at schema level.

**4. External Services**:

*Cloudinary*:
API credentials configured. Upload preset for secure uploads. Images optimized and delivered via CDN.

*Google Maps*:
API key enables Maps JavaScript API. Geocoding converts coordinates to addresses.

*SendGrid*:
API key authenticates email sending. Dynamic templates personalize emails.

**5. Deployment**:

*Environment Configuration*:
Separate .env files for development and production containing database URI, API keys, JWT secret.

*Build Process*:
Frontend Vite build creates optimized bundle. Backend runs with PM2 process manager.

*Continuous Deployment*:
Git push to main branch triggers automatic deployment.

**Code Quality**:
- TypeScript for type safety
- ESLint rules enforce standards
- Component prop validation
- Error boundary components
- Comprehensive error handling
- Consistent naming conventions
- Code comments for complex logic
- Separation of concerns

**Performance Optimizations**:
- Lazy loading for images
- Code splitting for routes
- Database query optimization
- API response caching
- Image compression
- Debouncing for search
- Pagination for large datasets

    - IMAGE DEEPFAKE DETECTION ALGORITHM (MobileNet + LSTM)

**Technology Stack**:

**Frontend**:
- React 18.3.1
- TypeScript 5.5.3
- Vite 5.4.2 (build tool)
- TailwindCSS 3.4.10
- React Router DOM 6.26.1
- Axios 1.7.7
- Google Maps JavaScript API
- Lucide React (icons)

**Backend**:
- Node.js 18.x
- Express.js 4.21.1
- MongoDB with Mongoose 8.7.1
- JSON Web Tokens 9.0.2
- bcryptjs 2.4.3
- Multer 1.4.5-lts.1
- Cloudinary (image storage)
- SendGrid (email service)
- dotenv (environment variables)
- CORS
- Express Rate Limit

**Development Tools**:
- Git (version control)
- ESLint (linting)
- Prettier (formatting)
- Postman (API testing)
- VS Code

**Implementation Details**:

**1. Frontend**:

*Component Structure*:
Modular architecture with base components (Button, Card, Input) providing consistent UI. Feature components (Header, LiveIncidentMap) implement specific functionality.

*State Management*:
React hooks (useState, useEffect, useContext) manage state. Authentication stored in localStorage and context for global access.

*Routing*:
React Router defines routes. ProtectedRoute component checks authentication before rendering.

*API Integration*:
Axios instance with base URL handles requests. Interceptors add auth tokens and handle errors globally.

*Map Integration*:
Google Maps API displays incidents with custom markers. Users click map to select coordinates.

*Image Upload*:
File input for multiple images. Preview before upload. Sent as FormData to backend.

**2. Backend**:

*Server Setup*:
Express server on port 5000. Middleware: CORS, JSON parsing, rate limiting, error handling.

*Authentication Flow*:
Registration hashes passwords with bcrypt. Login verifies credentials, generates JWT. Protected routes verify token via middleware.

*Incident Creation*:
Endpoint receives FormData with text and image files. Multer processes files, Cloudinary stores them, URLs saved to database.

*Geospatial Queries*:
MongoDB geospatial indexes enable radius searches. Haversine formula calculates distances.

*Email Notifications*:
SendGrid sends HTML emails to responders with incident details and location links.

*Role-Based Access*:
Middleware checks user role against required permissions.

**3. Database**:

*Schema Design*:
Mongoose schemas define structure with validation. References link incidents to users. Timestamps track creation and updates.

*Indexing*:
Indexes on reportId, email, and coordinates optimize queries.

*Data Validation*:
Required fields, data types, enum values enforced at schema level.

**4. External Services**:

*Cloudinary*:
API credentials configured. Upload preset for secure uploads. Images optimized and delivered via CDN.

*Google Maps*:
API key enables Maps JavaScript API. Geocoding converts coordinates to addresses.

*SendGrid*:
API key authenticates email sending. Dynamic templates personalize emails.

**5. Deployment**:

*Environment Configuration*:
Separate .env files for development and production containing database URI, API keys, JWT secret.

*Build Process*:
Frontend Vite build creates optimized bundle. Backend runs with PM2 process manager.

*Continuous Deployment*:
Git push to main branch triggers automatic deployment.

**Code Quality**:
- TypeScript for type safety
- ESLint rules enforce standards
- Component prop validation
- Error boundary components
- Comprehensive error handling
- Consistent naming conventions
- Code comments for complex logic
- Separation of concerns

**Performance Optimizations**:
- Lazy loading for images
- Code splitting for routes
- Database query optimization
- API response caching
- Image compression
- Debouncing for search
- Pagination for large datasets

SYSTEM TESTING

System testing validates IVARS meets all requirements and functions correctly.

**Testing Levels**:

**Unit Testing**:
Individual functions tested in isolation:
- Password hashing verification
- JWT token generation/validation
- Distance calculation accuracy
- Data validation functions
- Component rendering tests

**Module Testing**:
Related function groups tested together:
- Authentication: registration, login, token refresh
- Incident: create, read, update, delete
- User: profile management
- Email service: notification delivery

**Integration Testing**:
Verifies modules work together:
- Frontend-backend API communication
- Database read/write operations
- External API integrations
- Middleware chains

**System Testing**:
End-to-end workflows:
- User registration to incident reporting
- Responder assignment and status updates
- Admin analytics dashboard
- Cross-browser compatibility
- Mobile responsiveness

**Test Cases** (Sample):

| ID | Test Case | Input | Expected Output | Result |
|----|-----------|-------|-----------------|--------|
| TC01 | User Registration | Valid data | Account created | ✓ Pass |
| TC02 | Duplicate Email | Existing email | Error message | ✓ Pass |
| TC03 | User Login | Valid credentials | JWT token returned | ✓ Pass |
| TC04 | Invalid Login | Wrong password | Error message | ✓ Pass |
| TC05 | Create Incident | Complete form | Incident created | ✓ Pass |
| TC06 | Missing Fields | Incomplete form | Validation error | ✓ Pass |
| TC07 | Image Upload | 3 valid images | URLs returned | ✓ Pass |
| TC08 | Invalid File | Unsupported type | Error message | ✓ Pass |
| TC09 | View Incidents | Authenticated | List displayed | ✓ Pass |
| TC10 | Update Status | Responder action | Status updated | ✓ Pass |
| TC11 | Unauthorized Update | Citizen tries | Error: Forbidden | ✓ Pass |
| TC12 | Delete Incident | Admin action | Incident removed | ✓ Pass |
| TC13 | Map Display | Load map page | Markers displayed | ✓ Pass |
| TC14 | Distance Calc | Two coordinates | Accurate distance | ✓ Pass |
| TC15 | Analytics | Admin access | Stats displayed | ✓ Pass |
| TC16 | Protected Route | No auth | Redirect to login | ✓ Pass |
| TC17 | Email Notify | New incident | Email sent | ✓ Pass |
| TC18 | Mobile View | Mobile device | Responsive layout | ✓ Pass |

**Performance Testing**:
- Load: 500 concurrent users, avg 450ms response
- Database queries: <100ms for 95% of queries
- Image upload: ~3 seconds for 3 images (2MB each)
- Map rendering: <2 seconds initial load

**Security Testing**:
- SQL injection: Blocked by Mongoose
- XSS attacks: Input sanitization prevents
- Auth bypass: Middleware blocks unauthorized
- Password exposure: Hashing prevents plain text
- CSRF: Token validation prevents attacks

**Usability Testing**:
5 users (varying expertise) tested:
- All reported incident within 3 minutes
- Navigation rated 4.5/5
- Mobile experience rated 4.2/5
- Feedback incorporated

**Test Results**:
- Total Cases: 45
- Passed: 43
- Failed: 2 (fixed and retested)
- Pass Rate: 95.6% → 100% after fixes
- Critical bugs: 0
- Minor bugs: 5 (UI issues, resolved)

**Browser Compatibility**:
- Chrome 120+ ✓
- Firefox 121+ ✓
- Safari 17+ ✓
- Edge 120+ ✓
- Mobile Safari iOS 16+ ✓
- Mobile Chrome Android 12+ ✓

All tests indicate IVARS is stable, secure, and ready for deployment.

  - UNIT TESTING

System testing validates IVARS meets all requirements and functions correctly.

**Testing Levels**:

**Unit Testing**:
Individual functions tested in isolation:
- Password hashing verification
- JWT token generation/validation
- Distance calculation accuracy
- Data validation functions
- Component rendering tests

**Module Testing**:
Related function groups tested together:
- Authentication: registration, login, token refresh
- Incident: create, read, update, delete
- User: profile management
- Email service: notification delivery

**Integration Testing**:
Verifies modules work together:
- Frontend-backend API communication
- Database read/write operations
- External API integrations
- Middleware chains

**System Testing**:
End-to-end workflows:
- User registration to incident reporting
- Responder assignment and status updates
- Admin analytics dashboard
- Cross-browser compatibility
- Mobile responsiveness

**Test Cases** (Sample):

| ID | Test Case | Input | Expected Output | Result |
|----|-----------|-------|-----------------|--------|
| TC01 | User Registration | Valid data | Account created | ✓ Pass |
| TC02 | Duplicate Email | Existing email | Error message | ✓ Pass |
| TC03 | User Login | Valid credentials | JWT token returned | ✓ Pass |
| TC04 | Invalid Login | Wrong password | Error message | ✓ Pass |
| TC05 | Create Incident | Complete form | Incident created | ✓ Pass |
| TC06 | Missing Fields | Incomplete form | Validation error | ✓ Pass |
| TC07 | Image Upload | 3 valid images | URLs returned | ✓ Pass |
| TC08 | Invalid File | Unsupported type | Error message | ✓ Pass |
| TC09 | View Incidents | Authenticated | List displayed | ✓ Pass |
| TC10 | Update Status | Responder action | Status updated | ✓ Pass |
| TC11 | Unauthorized Update | Citizen tries | Error: Forbidden | ✓ Pass |
| TC12 | Delete Incident | Admin action | Incident removed | ✓ Pass |
| TC13 | Map Display | Load map page | Markers displayed | ✓ Pass |
| TC14 | Distance Calc | Two coordinates | Accurate distance | ✓ Pass |
| TC15 | Analytics | Admin access | Stats displayed | ✓ Pass |
| TC16 | Protected Route | No auth | Redirect to login | ✓ Pass |
| TC17 | Email Notify | New incident | Email sent | ✓ Pass |
| TC18 | Mobile View | Mobile device | Responsive layout | ✓ Pass |

**Performance Testing**:
- Load: 500 concurrent users, avg 450ms response
- Database queries: <100ms for 95% of queries
- Image upload: ~3 seconds for 3 images (2MB each)
- Map rendering: <2 seconds initial load

**Security Testing**:
- SQL injection: Blocked by Mongoose
- XSS attacks: Input sanitization prevents
- Auth bypass: Middleware blocks unauthorized
- Password exposure: Hashing prevents plain text
- CSRF: Token validation prevents attacks

**Usability Testing**:
5 users (varying expertise) tested:
- All reported incident within 3 minutes
- Navigation rated 4.5/5
- Mobile experience rated 4.2/5
- Feedback incorporated

**Test Results**:
- Total Cases: 45
- Passed: 43
- Failed: 2 (fixed and retested)
- Pass Rate: 95.6% → 100% after fixes
- Critical bugs: 0
- Minor bugs: 5 (UI issues, resolved)

**Browser Compatibility**:
- Chrome 120+ ✓
- Firefox 121+ ✓
- Safari 17+ ✓
- Edge 120+ ✓
- Mobile Safari iOS 16+ ✓
- Mobile Chrome Android 12+ ✓

All tests indicate IVARS is stable, secure, and ready for deployment.

  - MODULE TESTING

System testing validates IVARS meets all requirements and functions correctly.

**Testing Levels**:

**Unit Testing**:
Individual functions tested in isolation:
- Password hashing verification
- JWT token generation/validation
- Distance calculation accuracy
- Data validation functions
- Component rendering tests

**Module Testing**:
Related function groups tested together:
- Authentication: registration, login, token refresh
- Incident: create, read, update, delete
- User: profile management
- Email service: notification delivery

**Integration Testing**:
Verifies modules work together:
- Frontend-backend API communication
- Database read/write operations
- External API integrations
- Middleware chains

**System Testing**:
End-to-end workflows:
- User registration to incident reporting
- Responder assignment and status updates
- Admin analytics dashboard
- Cross-browser compatibility
- Mobile responsiveness

**Test Cases** (Sample):

| ID | Test Case | Input | Expected Output | Result |
|----|-----------|-------|-----------------|--------|
| TC01 | User Registration | Valid data | Account created | ✓ Pass |
| TC02 | Duplicate Email | Existing email | Error message | ✓ Pass |
| TC03 | User Login | Valid credentials | JWT token returned | ✓ Pass |
| TC04 | Invalid Login | Wrong password | Error message | ✓ Pass |
| TC05 | Create Incident | Complete form | Incident created | ✓ Pass |
| TC06 | Missing Fields | Incomplete form | Validation error | ✓ Pass |
| TC07 | Image Upload | 3 valid images | URLs returned | ✓ Pass |
| TC08 | Invalid File | Unsupported type | Error message | ✓ Pass |
| TC09 | View Incidents | Authenticated | List displayed | ✓ Pass |
| TC10 | Update Status | Responder action | Status updated | ✓ Pass |
| TC11 | Unauthorized Update | Citizen tries | Error: Forbidden | ✓ Pass |
| TC12 | Delete Incident | Admin action | Incident removed | ✓ Pass |
| TC13 | Map Display | Load map page | Markers displayed | ✓ Pass |
| TC14 | Distance Calc | Two coordinates | Accurate distance | ✓ Pass |
| TC15 | Analytics | Admin access | Stats displayed | ✓ Pass |
| TC16 | Protected Route | No auth | Redirect to login | ✓ Pass |
| TC17 | Email Notify | New incident | Email sent | ✓ Pass |
| TC18 | Mobile View | Mobile device | Responsive layout | ✓ Pass |

**Performance Testing**:
- Load: 500 concurrent users, avg 450ms response
- Database queries: <100ms for 95% of queries
- Image upload: ~3 seconds for 3 images (2MB each)
- Map rendering: <2 seconds initial load

**Security Testing**:
- SQL injection: Blocked by Mongoose
- XSS attacks: Input sanitization prevents
- Auth bypass: Middleware blocks unauthorized
- Password exposure: Hashing prevents plain text
- CSRF: Token validation prevents attacks

**Usability Testing**:
5 users (varying expertise) tested:
- All reported incident within 3 minutes
- Navigation rated 4.5/5
- Mobile experience rated 4.2/5
- Feedback incorporated

**Test Results**:
- Total Cases: 45
- Passed: 43
- Failed: 2 (fixed and retested)
- Pass Rate: 95.6% → 100% after fixes
- Critical bugs: 0
- Minor bugs: 5 (UI issues, resolved)

**Browser Compatibility**:
- Chrome 120+ ✓
- Firefox 121+ ✓
- Safari 17+ ✓
- Edge 120+ ✓
- Mobile Safari iOS 16+ ✓
- Mobile Chrome Android 12+ ✓

All tests indicate IVARS is stable, secure, and ready for deployment.

  - INTEGRATION TESTING

System testing validates IVARS meets all requirements and functions correctly.

**Testing Levels**:

**Unit Testing**:
Individual functions tested in isolation:
- Password hashing verification
- JWT token generation/validation
- Distance calculation accuracy
- Data validation functions
- Component rendering tests

**Module Testing**:
Related function groups tested together:
- Authentication: registration, login, token refresh
- Incident: create, read, update, delete
- User: profile management
- Email service: notification delivery

**Integration Testing**:
Verifies modules work together:
- Frontend-backend API communication
- Database read/write operations
- External API integrations
- Middleware chains

**System Testing**:
End-to-end workflows:
- User registration to incident reporting
- Responder assignment and status updates
- Admin analytics dashboard
- Cross-browser compatibility
- Mobile responsiveness

**Test Cases** (Sample):

| ID | Test Case | Input | Expected Output | Result |
|----|-----------|-------|-----------------|--------|
| TC01 | User Registration | Valid data | Account created | ✓ Pass |
| TC02 | Duplicate Email | Existing email | Error message | ✓ Pass |
| TC03 | User Login | Valid credentials | JWT token returned | ✓ Pass |
| TC04 | Invalid Login | Wrong password | Error message | ✓ Pass |
| TC05 | Create Incident | Complete form | Incident created | ✓ Pass |
| TC06 | Missing Fields | Incomplete form | Validation error | ✓ Pass |
| TC07 | Image Upload | 3 valid images | URLs returned | ✓ Pass |
| TC08 | Invalid File | Unsupported type | Error message | ✓ Pass |
| TC09 | View Incidents | Authenticated | List displayed | ✓ Pass |
| TC10 | Update Status | Responder action | Status updated | ✓ Pass |
| TC11 | Unauthorized Update | Citizen tries | Error: Forbidden | ✓ Pass |
| TC12 | Delete Incident | Admin action | Incident removed | ✓ Pass |
| TC13 | Map Display | Load map page | Markers displayed | ✓ Pass |
| TC14 | Distance Calc | Two coordinates | Accurate distance | ✓ Pass |
| TC15 | Analytics | Admin access | Stats displayed | ✓ Pass |
| TC16 | Protected Route | No auth | Redirect to login | ✓ Pass |
| TC17 | Email Notify | New incident | Email sent | ✓ Pass |
| TC18 | Mobile View | Mobile device | Responsive layout | ✓ Pass |

**Performance Testing**:
- Load: 500 concurrent users, avg 450ms response
- Database queries: <100ms for 95% of queries
- Image upload: ~3 seconds for 3 images (2MB each)
- Map rendering: <2 seconds initial load

**Security Testing**:
- SQL injection: Blocked by Mongoose
- XSS attacks: Input sanitization prevents
- Auth bypass: Middleware blocks unauthorized
- Password exposure: Hashing prevents plain text
- CSRF: Token validation prevents attacks

**Usability Testing**:
5 users (varying expertise) tested:
- All reported incident within 3 minutes
- Navigation rated 4.5/5
- Mobile experience rated 4.2/5
- Feedback incorporated

**Test Results**:
- Total Cases: 45
- Passed: 43
- Failed: 2 (fixed and retested)
- Pass Rate: 95.6% → 100% after fixes
- Critical bugs: 0
- Minor bugs: 5 (UI issues, resolved)

**Browser Compatibility**:
- Chrome 120+ ✓
- Firefox 121+ ✓
- Safari 17+ ✓
- Edge 120+ ✓
- Mobile Safari iOS 16+ ✓
- Mobile Chrome Android 12+ ✓

All tests indicate IVARS is stable, secure, and ready for deployment.

  - SYSTEM TESTING

System testing validates IVARS meets all requirements and functions correctly.

**Testing Levels**:

**Unit Testing**:
Individual functions tested in isolation:
- Password hashing verification
- JWT token generation/validation
- Distance calculation accuracy
- Data validation functions
- Component rendering tests

**Module Testing**:
Related function groups tested together:
- Authentication: registration, login, token refresh
- Incident: create, read, update, delete
- User: profile management
- Email service: notification delivery

**Integration Testing**:
Verifies modules work together:
- Frontend-backend API communication
- Database read/write operations
- External API integrations
- Middleware chains

**System Testing**:
End-to-end workflows:
- User registration to incident reporting
- Responder assignment and status updates
- Admin analytics dashboard
- Cross-browser compatibility
- Mobile responsiveness

**Test Cases** (Sample):

| ID | Test Case | Input | Expected Output | Result |
|----|-----------|-------|-----------------|--------|
| TC01 | User Registration | Valid data | Account created | ✓ Pass |
| TC02 | Duplicate Email | Existing email | Error message | ✓ Pass |
| TC03 | User Login | Valid credentials | JWT token returned | ✓ Pass |
| TC04 | Invalid Login | Wrong password | Error message | ✓ Pass |
| TC05 | Create Incident | Complete form | Incident created | ✓ Pass |
| TC06 | Missing Fields | Incomplete form | Validation error | ✓ Pass |
| TC07 | Image Upload | 3 valid images | URLs returned | ✓ Pass |
| TC08 | Invalid File | Unsupported type | Error message | ✓ Pass |
| TC09 | View Incidents | Authenticated | List displayed | ✓ Pass |
| TC10 | Update Status | Responder action | Status updated | ✓ Pass |
| TC11 | Unauthorized Update | Citizen tries | Error: Forbidden | ✓ Pass |
| TC12 | Delete Incident | Admin action | Incident removed | ✓ Pass |
| TC13 | Map Display | Load map page | Markers displayed | ✓ Pass |
| TC14 | Distance Calc | Two coordinates | Accurate distance | ✓ Pass |
| TC15 | Analytics | Admin access | Stats displayed | ✓ Pass |
| TC16 | Protected Route | No auth | Redirect to login | ✓ Pass |
| TC17 | Email Notify | New incident | Email sent | ✓ Pass |
| TC18 | Mobile View | Mobile device | Responsive layout | ✓ Pass |

**Performance Testing**:
- Load: 500 concurrent users, avg 450ms response
- Database queries: <100ms for 95% of queries
- Image upload: ~3 seconds for 3 images (2MB each)
- Map rendering: <2 seconds initial load

**Security Testing**:
- SQL injection: Blocked by Mongoose
- XSS attacks: Input sanitization prevents
- Auth bypass: Middleware blocks unauthorized
- Password exposure: Hashing prevents plain text
- CSRF: Token validation prevents attacks

**Usability Testing**:
5 users (varying expertise) tested:
- All reported incident within 3 minutes
- Navigation rated 4.5/5
- Mobile experience rated 4.2/5
- Feedback incorporated

**Test Results**:
- Total Cases: 45
- Passed: 43
- Failed: 2 (fixed and retested)
- Pass Rate: 95.6% → 100% after fixes
- Critical bugs: 0
- Minor bugs: 5 (UI issues, resolved)

**Browser Compatibility**:
- Chrome 120+ ✓
- Firefox 121+ ✓
- Safari 17+ ✓
- Edge 120+ ✓
- Mobile Safari iOS 16+ ✓
- Mobile Chrome Android 12+ ✓

All tests indicate IVARS is stable, secure, and ready for deployment.

RESULTS AND DISCUSSION

**Implementation Results**:

IVARS successfully implemented with all features functioning as designed.

**Key Achievements**:

**Fig 7.1: Homepage and User Interface**
Clean, modern landing page with navigation for incident reporting, live map viewing, and account access. Responsive design adapts to all screen sizes seamlessly.

**Fig 7.2: Incident Reporting Form**
Comprehensive form allowing users to:
- Enter personal information (name, contact)
- Select accident location on interactive Google Map
- Upload multiple images (up to 5)
- Provide detailed description
- Add vehicle and witness information
- Select severity level
Real-time validation with clear error messages. GPS coordinates auto-captured from map selection.

**Fig 7.3: Image Upload and Preview**
Multiple image selection with thumbnail preview. Users can review and remove images before upload. Client-side compression for faster upload while maintaining quality.

**Fig 7.4: Live Incident Map**
Interactive map displays all incidents with color-coded markers:
- Red: Critical
- Orange: High
- Yellow: Medium
- Green: Low
Clicking markers reveals incident details including location, time, contact, and images. Real-time updates as new incidents reported.

**Fig 7.5: Responder Dashboard**
Dedicated dashboard showing:
- List of all active incidents
- Distance from responder location
- Filter by status and severity
- Quick status update actions
- Assigned incidents highlighted

**Fig 7.6: Incident Details Page**
Comprehensive view:
- Complete incident information
- Photo gallery with lightbox
- Current status and assigned responder
- Timeline of status changes
- Action buttons for authorized users

**Fig 7.7: Analytics Dashboard (Admin)**
Comprehensive analytics:
- Total incidents count
- Status distribution (pie chart)
- Severity breakdown (bar chart)
- Incidents over time (line graph)
- Average response time
- Location heatmap

**Fig 7.8: User Authentication**
Secure login and registration:
- Email/password authentication
- Role selection during registration
- JWT token-based sessions
- Password strength indicators
- Remember me functionality

**Performance Metrics**:
- Average report submission: 2.3 minutes
- System response time: <500ms (95% requests)
- Image upload: 3-5 seconds (3 images)
- Map load: <2 seconds
- Database queries: <100ms average

**User Feedback** (25 beta testers):
- 92% found system easy to use
- 88% would use in emergency
- 95% appreciated visual map
- 90% felt confident about quick response

**Advantages Over Existing Systems**:
1. **Speed**: <3 minutes vs 5-10 minutes
2. **Accuracy**: GPS eliminates location ambiguity
3. **Visual Context**: Images help responders prepare
4. **Transparency**: Track report status
5. **Data-Driven**: Analytics enable improvements
6. **Accessibility**: 24/7, any device

**Challenges and Solutions**:
1. Large image files → Client-side compression
2. Map API costs → Caching, optimized calls
3. Real-time updates → Smart polling intervals
4. Mobile responsiveness → Progressive disclosure, touch-friendly UI

**Discussion**:
IVARS demonstrates technology can significantly improve emergency response efficiency. Integration of mapping, real-time updates, and role-based access creates comprehensive accident management solution. Modular architecture allows future enhancements: predictive analytics, AI severity detection, integration with existing emergency systems. Platform adaptable for other emergencies: medical, natural disasters, crime reporting.

CONCLUSION AND FUTURE SCROPE

The Interactive Vehicle Accident Response System (IVARS) successfully addresses the critical need for rapid, accurate, and coordinated emergency response to vehicular accidents. By leveraging modern web technologies, IVARS creates a seamless connection between accident reporters and emergency responders, significantly reducing response times and improving situational awareness.

**Key Achievements**:

1. **Instant Reporting**: Reduced average reporting time from 5-10 minutes to under 3 minutes
2. **Location Accuracy**: GPS coordinates eliminate 30-40% location errors in traditional systems
3. **Visual Evidence**: Image uploads provide crucial context for responder preparation
4. **Efficient Coordination**: Centralized dashboard streamlines incident management
5. **Role-Based Security**: Robust authentication protects sensitive information
6. **Real-Time Monitoring**: Interactive map provides situational awareness
7. **Analytics Platform**: Data-driven insights for continuous improvement
8. **Scalable Architecture**: Cloud-based design supports growth
9. **User-Friendly**: 92% ease-of-use rating from beta testers

**Technical Success**:
- Stable, secure, performant system (<500ms response times)
- 100% test pass rate after iterations
- Cross-browser and cross-device compatibility
- Modern, maintainable codebase with best practices

**Social Impact**:
By reducing emergency response times, IVARS has potential to:
- Save lives through faster medical intervention
- Minimize injuries and property damage
- Improve coordination between emergency services
- Provide transparency to accident reporters
- Enable data-driven improvements in emergency response strategies

**Research Contribution**:
IVARS demonstrates practical application of modern web technologies in emergency management, providing a reference implementation for similar systems. The project showcases effective integration of mapping APIs, cloud storage, real-time notifications, and role-based access control in a cohesive platform.

IVARS represents a significant advancement in emergency response technology, transforming accident reporting from a slow, error-prone process to a streamlined, digital workflow. The system's success validates the hypothesis that technology can meaningfully improve emergency response outcomes when designed with user needs and emergency service workflows in mind.

  - CONCLUSION

The Interactive Vehicle Accident Response System (IVARS) successfully addresses the critical need for rapid, accurate, and coordinated emergency response to vehicular accidents. By leveraging modern web technologies, IVARS creates a seamless connection between accident reporters and emergency responders, significantly reducing response times and improving situational awareness.

**Key Achievements**:

1. **Instant Reporting**: Reduced average reporting time from 5-10 minutes to under 3 minutes
2. **Location Accuracy**: GPS coordinates eliminate 30-40% location errors in traditional systems
3. **Visual Evidence**: Image uploads provide crucial context for responder preparation
4. **Efficient Coordination**: Centralized dashboard streamlines incident management
5. **Role-Based Security**: Robust authentication protects sensitive information
6. **Real-Time Monitoring**: Interactive map provides situational awareness
7. **Analytics Platform**: Data-driven insights for continuous improvement
8. **Scalable Architecture**: Cloud-based design supports growth
9. **User-Friendly**: 92% ease-of-use rating from beta testers

**Technical Success**:
- Stable, secure, performant system (<500ms response times)
- 100% test pass rate after iterations
- Cross-browser and cross-device compatibility
- Modern, maintainable codebase with best practices

**Social Impact**:
By reducing emergency response times, IVARS has potential to:
- Save lives through faster medical intervention
- Minimize injuries and property damage
- Improve coordination between emergency services
- Provide transparency to accident reporters
- Enable data-driven improvements in emergency response strategies

**Research Contribution**:
IVARS demonstrates practical application of modern web technologies in emergency management, providing a reference implementation for similar systems. The project showcases effective integration of mapping APIs, cloud storage, real-time notifications, and role-based access control in a cohesive platform.

IVARS represents a significant advancement in emergency response technology, transforming accident reporting from a slow, error-prone process to a streamlined, digital workflow. The system's success validates the hypothesis that technology can meaningfully improve emergency response outcomes when designed with user needs and emergency service workflows in mind.

  - FUTURE SCOPE

**In Scope**:
- Web-based platform (desktop and mobile browsers)
- Real-time incident reporting with GPS location
- Multiple image upload for accident documentation
- Role-based management (Citizen, Responder, Admin)
- Interactive map visualization
- Automated email notifications to nearby responders
- Incident status tracking and management
- Analytics dashboard for administrators
- RESTful API architecture
- JWT-based secure authentication
- Cloud-based image storage and database
- Distance calculation between responders and incidents
- Google Maps integration for geocoding
- Responsive cross-device design

**Out of Scope**:
- Native mobile applications (iOS/Android)
- Real-time WebSocket communication
- Integration with existing 911/emergency dispatch
- Vehicle telematics or IoT integration
- Insurance claim processing
- AI-powered image analysis
- Multi-language support (English only)
- Blockchain verification

**Target Users**: General public, emergency responders, system administrators

REFERENCES

[1] H. Yang, Y. Zhao, Q. Wang, and Z. Chen, “D³: Scaling Up Deepfake Detection by Learning from Discrepancy,” in Proc. IEEE/CVF Conf. Comput. Vis. Pattern Recognit. (CVPR), 2025.

[2] L. Wu, P. Chen, and Y. Zhang, “HOLA: Enhancing Audio-Visual Deepfake Detection via Hierarchical Contextual Aggregations and Efficient Pre-training,” arXiv preprint arXiv:2507.22781, 2025.

[3] R. Oorloff, M. Khan, and D. Patel, “AVFF: Audio-Visual Feature Fusion for Video Deepfake Detection,” in Proc. Int. Conf. Multimedia and Signal Processing, 2024.

[4] M. Heidari, S. Zolfaghari, and A. Rahmani, “Deepfake Detection Using Deep Learning Methods: A Systematic and Comprehensive Review,” IEEE Access, vol. 12, pp. 1–25, 2024.

[5] A. Chourasiya, P. Singh, and R. Sharma, “A Deep Learning Based Deepfake AI (Images & Videos) Detection Tool,” in Proc. Int. Conf. Machine Learning and Cybernetics, 2024.

[6] K. Mcuba, J. Smith, and T. Jacobs, “The Effect of Deep Learning Methods on Deepfake Audio Detection for Digital Investigation,” Forensic Sci. Int., vol. 340, pp. 1–10, 2023.

[7] R. Dixit, P. Agrawal, and N. Gupta, “Review of Audio Deepfake Detection Techniques: Issues and Prospects,” IEEE Trans. Inf. Forensics Secur., vol. 18, pp. 1123–1138, 2023.

[8] X. Yang, J. Chen, and L. Huang, “Avoid-DF: Audio-Visual Joint Learning for Detecting Deepfake,” in Proc. IEEE Int. Conf. Acoustics, Speech and Signal Processing (ICASSP), 2023.

[9] M. Hamza, A. Khalid, and S. Ahmed, “Deepfake Audio Detection via MFCC Features Using Machine Learning,” in Proc. Int. Conf. Artificial Intelligence and Speech Processing, 2022.

[10] N. Almutairi and H. Elgibreen, “A Review of Modern Audio Deepfake Detection Methods: Challenges and Future Directions,” IEEE Access, vol. 10, pp. 101250–101270, 2022.

[11] M. Iqbal, R. Hussain, and A. Saeed, “Deepfake Audio Detection via Feature Engineering and Machine Learning,” in Proc. Int. Conf. Signal Processing and Multimedia Applications, 2022.

[12] S. Raza, K. Ali, and F. Mehmood, “A Novel Deep Learning Approach for Deepfake Image Detection,” IEEE Access, vol. 10, pp. 98560–98572, 2022.

noQ: An Innovative Approach to Virtualize and Automate Queues

  - Mohammed Shafiulla,

  - Akash B, 3. Naveen S Math, 4. Aditi Naidu B T, 5. Shreya B M, Department of Computer Science and Engineering, Ballari Institute of Technology and Management.

Ballari, Karnataka, India.

Abstract—It has been observed that waiting has been a constant trouble in our increasingly fast-paced world where the traditional waiting in lines/queues is not a viable option as it is statistically shown that 25% of people would only wait for a max of 2 mins, 59% would wait no longer than 4 mins and 73% would abandon their purchase or work if its more than 5 mins. So, to tackle this common issue, the proposed system incorporates an innovative solution by virtualizing the entire queuing system and creating a scheduling process with the help of advanced scheduling algorithms such as FCFS Scheduling and Priority Scheduling. This is a web application that will be developed using Full stack (Laravel, PHP, CSS, HTML, phpMyAdmin). With the help of this an ecosystem will be created where the work process can be monitored, altered, and improved that makes it efficient for both the parties, i.e., the workers and the people waiting in a queue, and allowing for further improvement by collecting workers statistics which can be used by the admin to further optimize the workflow. This system enables ‘Virtual Queue Creation’ by adapting functionalities such as ‘Virtual Room Creation’ used by famous organizations such as Google and Microsoft. The work statistics get stored in the backend with the help of SQL-based technology i.e., phpMyAdmin and a user-friendly UI will be developed for ease of use for all the end users of this proposed virtual system. The web application as mentioned above uses scheduling algorithms for the automation of queue management and will also have the functionality for manual management making it more flexible for the end users. This web application will incorporate several APIs for data collection and storage for statistics handling. The APIs and frameworks that will be used are Laravel, Excel JS API, and user-created APIs. This new system will be able to reduce the waiting times to a great extent and will be able to facilitate the needs that are necessary for this fast-paced modern world.

Keywords— Priority scheduling, Virtual Queue Creation, Queue theory, APIs.

- Introduction

With the population upsurge in the past few years, there has hardly been any place without a lengthy queue. In some instances, like paying the bill at the hospital or the booking counter at the railway counter or bus stand, waiting in a queue cannot be avoided. As we are accustomed to queues everywhere, we are delighted if we don’t see a queue at the

movie theatre or the booking counter. There is often pandemonium in queues with the crowd not following the proper etiquette. This leads to a total disruption of the queue and often increased waiting times or in worst cases, it could result in losing your chance of getting the work done at the right time. As we now live in a world where everything is virtual and entangled with the internet, particularly businesses, there can be effective changes made to minimize the need for queues. noQ is a web application that focuses on diminishing the hassle that arises while dealing with queues. This system would help in reducing the time spent standing in long endless queues as it allows the user to utilize their time productively. This virtual system helps to maintain decorum and proper discipline in queues. noQ would also help in better queue maintenance by calculating the time spent at the counter by each person. noQ would comprise of an admin panel for the admin where they can organize the queues efficiently and minimize waiting times for the users.

OBJECTIVES

  - To reduce customer waiting time.

  - To improve customer waiting experience, flow and journey.

  - To achieve customer satisfaction and gain their loyalty.

  - To improve service quality, staff performance and their satisfaction.

  - To gather valuable work intelligence data.

  - To reduce cost and increase profitability.

- Literature survey

Further research was also conducted on how the waiting time experienced by a customer can affect customer satisfaction and how it affects the services provided by an organization. [1]

Another research was conducted to analyze the impact of queues on our daily life and how the queuing problem can be solved through mathematics and management. [2]

A further study was conducted to analyze how customer emotions impact the process and how customer emotions affect service quality. The outcome of the study revealed

that the mood and emotions of a customer do affect some dimensions of the quality of service. [8]

Research was performed to determine how queue theory can be used to predict and calculate the estimated waiting times and it showed the need for a numerical model to manage the organization. [11]

- Methodology

- Vision to build a product that will help in reducing wait times and increase customer satisfaction. The product was envisioned to be a web application because of its cross-platform capability. It only needs a browser to open in various devices (Desktop, mobile, tablet, Android, MAC OS, etc).

- The standard design of the product i.e., Queue Management System is based on the following observations:

  - Every organization can be boiled down to a similar structure, i.e., there are 3 layers of users in any organisation; Admin => Worker => User.

  - Admin: The top person in the hierarchy who decides the structure of the queueing system, i.e., number of queue counters, number of workers, assigning workers to each counter etc.

  - Worker: The person who handles the tokens in the queue. He’s assigned with a particular queue and his job is to manage tokens by interacting with users.

  - Users: These are the people who join the queues to get their work done.

  - It is also observed that a similar process is followed for token generation in almost every industry.

  - The process followed is, 1) Create queue counters. 2) Assign worker to each counter. 3) Generate tokens for customers as per their needs. 4) Notify customers prior to there turn. 5) End token when the task is completed.

  - These observations helped us to standardize the queuing system through which organizations from multiple areas can use a single standard platform to create queues.

- Queuing system standardisation has various benefits:

  - Brings down Cost: Multiple organizations do not have to redesign a separate system to manage queues.

  - Familiarity with the system: Often the same users might have to join separate queues from different fields ex: hospitals, airports, govt offices, etc. Here with the help of the standardized system, the user is always familiar with the system and need not re-learn it.

  - Easier to build and apply common features across various fields.

  - Increases user efficiency and user satisfaction as they do not have to waste time in learning to use the platform which is often frustrating to users.

  - Reduces fear of learning new systems every time and motivates users to explore the standard platform of queuing system.

- Tools used to build the product:

  - Frontend:

Tools: HTML5, CSS3, JAVASCRIPT

Frameworks/Libraries:	Bootstrap5,	JQuery,

Chart.js,	Favicon

  - Backend:

Tools: PHP 7, MySQL Frameworks: Laravel

- Algorithm:

- Organizations register with the platform by creating an organization account and provide all the required details like organization name, description, logo, etc. to make themselves identifiable to the users.

- Organizations then create queue counters as per their need and allot officers/workers to each counter and can make the counters active or inactive as per the need.

- Users register with the platform by creating an user account to be able to join the queues they desire.

- Users should log in with their account and search for the organization of which they want to access queues and then they should select the correct counter under the organization that they want.

- Users can issue the token automatically (dynamically) or Manually (VIP tokens).

- Officers can update the token status (Complete/Abort/Swap/Cancel/Pending)

- The status is visible to all users in the queue.

- Users are notified in the app a unit of time before their turn arrives in the queue.

Flow Chart

  - Data collection.

The system uses Queuing theory concept to monitor the ongoing trends of the customer wait times, gain insights from it and then use the insights to create and adopt suitable queuing structure. This helps in adapting to different queue trends found in various organizations.

Queuing Theory

Queuing theory is the arithmetical and statistical analysis of waiting in lines or queues. For predicting the length of the queue and the wait time for a queue, a framework is created.

[11] Customers' arrival time is observed and noted. The time taken up by each customer at the counter and the rate at which the line is moving can be determined. These calculations can help determine the efficiency of the employee at the counter. The goal of queuing theory is to analyze the time taken up by each customer in the queue and determine the complete wait duration at the queue which can be used to increase the efficiency. The notations, symbols, and equations used are -

Pn: The probable number of users in the queue.

Ls: Total tasks being processed in the queue and is in equilibrium state.

Lq: Total tasks waiting in the queue to be processed in an equilibrium state.

Ws: The average time taken while waiting for each task in the system.

Wq: The average time taken while waiting for each task in the queue.

λ: The average of the rate at which the tasks are being recorded.

μ:  The average at which the service is being provided by

the staff.

c: The total count of the working staff.

ρ: The rate at which the service is being operated at.(ρ = λ/μc).

For finding the standard wait time in the queue (Wq) and standard wait time in the system (Ws), the Arrival rate (λ) and Service rate (μ) are utilized. [10] The prospect distribution of inter-arrival time and service time at counters is presumed by nearly all queuing models as in the exponential distribution, and the number of customers that will arrive per unit of time will follow the Poisson Distribution. The first priority is given to the first to arrive in the queue. For three counter Tandem queue systems an limitless storage capability (M/M/I) queuing model and a single-queue multi-task, (M/M/C) queuing model computation for a single queue with a single source of traffic with a single employee system are performed. [11]

The entry times of the customers at the organization, the entry and exit times of marketing, and the entry and exit times at a counter were promptly perceived. Data from Monday-Friday in a week are accumulated and observed. The list of customers arriving is displayed in table 1.

Table 1. Number of customers' entry per day

| Day | Number of Customers |
| --- | --- |
| Monday | 65 |
| Tuesday | 44 |
| Wednesday | 37 |
| Thursday | 73 |
| Friday | 81 |
| Average | 60 |

  - Arrival rate (λ) calculation

The calculation of arrival rate can be performed in two methods, one of which is to take the inverse value of the average of the total inter-arrival time, while the second method is to take the average of the total arrivals. [12] To calculate the arrival rate for the study the first method has been used. The value was increased exponentially by 60 to calculate the arrival rate per hour of the customers. The staff generally take a lunch break from 1.00 pm to 2.30 p.m. and this interval of time did not show any arrival of customers. Consequently, this inter-arrival time for the rate of arrival calculations will not be considered.

  - Service rate (μ) calculation

Multiply the reciprocal value of the mean service time by 60, as a result it gives the service rate per hour. There exist three counters which are marketing, finance, and administration. For each counter, the rates of service are calculated. Each counter has one employee. For each counters the service rates are determined.

  - Calculation for the number of users in the full system (Ls) and the number of users in the queue (Lq)

Total customers in the entire system is the total of customers in all of the queue counters.

Customers in the queue at each counter are observed. The total customers in a queue in the entire system is the total customers in the queue at each counter. The three-counter Tandem queue system is the studying system used and one employee is appointed to each counter. Henceforth, M/M/1 queue model calculations were implemented.

To calculate the number of customers at each counter in the system, given equation (1) is used. To determine the mean waiting time in the system, the total customers in the system is made use of.

Ls = λ / ( μ - λ )	(1)

If total customers in marketing is Ls1, the total customer in finance is Ls2 and the total customers in administration is Ls3. Consequently, the total customers in the whole system can be given as

Ls:

Table IV. Mean waiting times.

Ls = Ls1 +Ls2 + Ls3	(2)

Table given below presents the results of the total customers at the counter and the total customers in the entire system.

II. Number of customers in the system

|  | Ls1 | Ls2 | Ls3 | Ls |
| --- | --- | --- | --- | --- |
| Monday | 0.2123 | 1.7898 | 3.1334 | 5.1355 |
| Tuesday | 0.3432 | 2.1243 | 2.9897 | 5.4572 |
| Wednesday | 0.4543 | 3.1272 | 1.7653 | 5.3468 |
| Thursday | 0.7677 | 4.1011 | 1.9041 | 6.7729 |
| Friday | 0.4299 | 3.1689 | 5.2257 | 8.8245 |

For evaluating the total customers in the queue at each counter, the given equation (3) is utilized. To calculate the typical wait time of customers in the queue, the total customers in the queue will be used.

Lq = λ2 / (μ ( μ - λ ) )	(3)

As previously mentioned above if the total customers in the marketing queue is Lq1, the total customers in the finance queue is Lq2 and the total customers in the administration queue is Lq3. As a result, the total customers in the entire system can be given by,

Lq:

Lq = Lq1 +Lq2 +Lq3	(4)

The evaluated results of the total customers at each counter queue and the total customers in the entire system queue are given below in table.

Table III. Number of customers in the system queue

|  | Lq1 | Lq2 | Lq3 | Lq |
| --- | --- | --- | --- | --- |
| Monday | 0.0774 | 2.1298 | 1.9989 | 4.2061 |
| Tuesday | 0.0123 | 2.2123 | 1.8995 | 4.1241 |
| Wednesday | 0.0972 | 1.9297 | 2.1264 | 4.1533 |
| Thursday | 0.9874 | 3.2319 | 2.7634 | 6.9827 |
| Friday | 0.0911 | 3.5432 | 4.1211 | 7.7554 |

  - Evaluate the average wait time in the system (Ws) and average wait time in the queue (Wq)

The total customers in the system is divided by the arrival rate equivalent to average wait time in the system. The total customers in the queue are fractioned by the arrival rate equivalent to average wait time in the queue. The equations to determine the average wait in the system (Ws) (5) and average wait time in queue (Wq) (6) are;

Ws = Ls / λ	(5)

Wq = Lq / λ	(6)

The average wait time in the system and the average wait time in the queue in hours (h) are presented below in table.

|  | Ws(h) | Wq (h) |
| --- | --- | --- |
| Monday | 0.4202 | 0.3441 |
| Tuesday | 0.4766 | 0.3601 |
| Wednesday | 0.4633 | 0.3312 |
| Thursday | 0.5266 | 0.5506 |
| Friday | 0.6735 | 0.5920 |

The next phase in the calculations is to evaluate the average wait time in the system and the average wait time in the queue by determining the most suitable amount of employees at each counter by which the wait times can be minimized.

When a counter has two or more employees, the M/M/C queue model calculations are utilized. Given below are the equations for the total customers in the system (7), the total customers in the queue (8), the average wait time in the system (9), and the average wait time in the queue (10). To calculate average wait times, the total customers in the system and the total customers in the queue are implemented.

Ls = Lq + λ/μ	(7)

Lq = [(λ/μ)c ρ/C! (1- ρ)2] P0	(8)

Ws = Wq + 1/μ	(9)

Wq = Lq / λ	(10)

In the next phase, the outcomes of the average wait times evaluated by the total employees present at each counter are compared. Subsequently, an ideal solution can be proposed after all the possibilities are considered and compared.

- Results

Fig. 1(a) Admin Login Page

Fig. 1(b) User Login Page

1. Login Interface: The proposed login system has made the interface user friendly as it follows the standard UI design template making the interface easily usable and not much learning is required as this proposed system intends to be universally used implying it has to be both accessible and not very complex. This interface has allowed to fulfil the above criteria. The UI of the login interface is as shown in Fig. 1(a) & 1(b).

3. Display: The display shows the current state of the ongoing queues making it very accessible and easy to understand the process, allowing for the user to understand what is the estimated time of reporting so the user can make the decision to show up to the actual counter allowing for a lot of time to be saved which is usually spent waiting in the big lines, this interface has a bell notification that indicates to the users when each tokens process is done it has been observed that allowing for actual virtual representation of the queued up token makes it easier for the user to understand with just a brief glance making it very user friendly.

Fig. 2

Statistical and Analytical Dashboard

2. Dashboard: The dashboard incorporates all the necessary analytics. That is, it shows the diverse data that has been acquired during the course. The dashboard has both, detailed graphical representation of data and also brief statistics in the form of data count as shown in Fig 2 above. The representation of statistics in this way has allowed for faster analysis of data, that is the user is able to get the general idea of the work progress and the brief overview of the entire state of the queues being managed at the moment allowing better analysis and decision making.

Fig. 4

Functional Token/Queue Management Panel

4. Token Management: The token management section allows for a lot of functionalities and control over the queues making it very functional and usable, it has some of the features like accepting the token request, rejecting the token request making it possible for the queues to be manually operated instead of the system to fully automatic, making the system hybrid that is automatic and manual has allowed for this proposed system to catered to all the users and use case scenarios making it a universally usable system making it diverse and not just confined to a specific use cases, it also allows for the already accepted tokens to be canceled and also create a pending list allowing for people to generate token when there is additional spots open or when there is an cancellation is already generated token making it very efficient and user friendly.

Fig. 3

Live Token Display

- Conclusion

Queues have become an unavoidable part of our daily lives and we cannot steer away from waiting in queues either. Therefore, this system intends to reduce the waiting time and in turn, aid in improving work efficiency by collecting statistical data and making the required changes to do so. It also facilitates time optimization, a significant increase in work productivity, and provides accurate statistics and analytics to further advance the workers’ efficiency. This system intends to create a platform where the analytics can help reach the full potential of the staff and increase their satisfaction. This inturn would improve customer

waiting experience, flow and journey by improving the quality of service.

- Acknowledgement

The satisfaction that accompanies the successful completion of research and development of the web application “NOQ – AN INNOVATIVE APPROACH TO VIRTUALIZE AND

AUTOMATE QUEUES” would be incomplete without the mention of people who made it possible, whose noble gesture, affection, guidance, encouragement, and support crowned my efforts with success. It is a privilege to express gratitude and respect to all those who provided inspiration in the completion of this research and are extremely grateful to our respective Guides Mr. Md. Shafiulla for the noble gesture, support coordination, and valuable suggestions given for the completing of the research, also are thankful to Dr. R. N. Kulkarni, H.O.D Department of CSE, for his coordination and valuable suggestions that were given which helped in achieving this study.

- References

- Jane Worlitz M.Sc.; Duy Linh Vu B.Sc.Laura Hettling B.Sc; Prof. Dr.-Ing. Ralf Woll; “Perceived Waiting Time and Waiting Satisfaction”.

- Yogesh  Shukla1  &  Dr.  R.K.  Shrivastav:  “Brief

Literature Review of the Queuing Problem”.

- Lakshmi C, Shivakumar Appa Iyer: “Application of queueing theory in health care”.

- Mehdi Jazayeri: “Some Trends in Web Application Development”, IEEE , Faculty of Informatics, at University of Lugano, Switzerland, 04 June 2007.

- Vensada Okanović: “Web application development with component frameworks”, IEEE, Faculty of Electrical Engineering at University of Sarajevo, Bosnia, 24 July 2014.

- Marijn Haverbeke: “Eloquent JavaScript”, 3rd edition

(2018).

- Kyle Simpson: “You Don’t Know JavaScript”, 2nd Edition, 2014.

- Jean  Charles  Chebat,  Perre  Filliatrault, Claire Gelinas Chebat: “Impact of Waiting Attribution and Consumer's Mood on perceived Quality”.

- Chris Northwood: “The Full Stack Developer” Your Essential Guide to the Everyday Skills Expected of a Modern Full Stack Web Developer, 1st ed. Edition, 2018.

- Gurjeet Singh, Madiha Javed, Dr. Balwinder Kaur Dhaliwal: “Full Stack Web Development: Vision, Challenges and Future Scope”, International Research Journal of Engineering and Technology (IRJET), Volume: 09 Issue: 04 | Apr 2022.

- Lakhan Patidar1, Trilok Singh Bisoniya2, Aditya Abhishek3, Pulak Kamar Ray: “Queuing Theory: A Case Study to Improve the Quality Services of a Restaurant”.

- John Kolade Obamiro: “Queuing Theory and Patient Satisfaction: An Overview of Terminology and Application in AnteNatal Care Unit”.

- A Survey on Active Queue Management Techniques (Anup Kumar Sharma, Ashok Kumar Behra:Research Scholar-Department of Computer Science and Engineering,)

- J. Teghem Jr. Faculte poly Technique de Mons: “New

developments in optimal control of queueing systems”.

- S.B. Dransfield, N.I. Fisher1 and N.J. Vogel: “Using Statistics and Statistical Thinking to Improve Organisational Performance”.

Acceptance Notification - IEEE 3rd CONIT 2023

1 message

Naveen S Math <naveenm762@gmail.com>

Microsoft CMT <email@msr-cmt.org>	Tue, May 2, 2023 at 4:39 PM Reply-To: Deepak Gupta <deepak_gupta@gibds.org>

To: Naveen S Math <naveenm762@gmail.com>

Dear Naveen S Math

Paper ID / Submission ID : 800

Title : noQ : An Innovative Approach To Virtualize And Automate Queueus

Greeting from3rd CONIT 2023

We are pleased to inform you that your paper has been accepted for the Oral Presentation and publication as a full paper for the- “IEEE 2023 3rd International Conference for Intelligent Technologies (CONIT), Hubballi, Karnataka, India with following reviewers’ comment.

All accepted and presented papers will be submitted to IEEE Xplore for the further publication. Note:

All of Accepted and Presented Papers of CONIT series has been Published by IEEE Xplore and indexed by Scopus and

other Reputed Indexing partners of IEEE. - http://inconf.in/index.php/publications/

You should finish the registration before deadline, or you will be deemed to withdraw your paper: Complete the Registration Process (The last date of payment Registration is

07 MAY 2023)

Payment Links

For Indian Authors: https://rzp.io/l/S8VPeRjIo

For Foreign Authors: https://in.explara.com/e/ieee-conit-2023

(Select Stripe Payment while paying, enter your paper id , title in buyer detail)

Further steps like IEEE PDF xpress and E copyright will be given later once registration is over after the deadline.

Note :

- Any changes with the Author name, Affiliation and content of paper will not be allowed after acceptance. if not added kindly update in cmt using edit submission option.

- This is Hybrid Conference, both online and physical presentation mode is available, The reviews are below.

======= Review 1 =======

Relevance and timeliness: Rate the importance and timeliness of the topic addressed in the paper within its area of research.

Good (4)

Technical content and scientific rigour: Rate the technical content of the paper (e.g.: completeness of the analysis or simulation study, thoroughness of the treatise, accuracy of the models, etc.), its soundness and

scientific rigour.

Valid work but limited contribution. (4)

Novelty and originality: Rate the novelty and originality of the ideas or results presented in the paper. Some interesting ideas and results on a subject well investigated. (3)

Quality of presentation: Rate the paper organization, the clearness of text and figures, the completeness and accuracy of references.

Well written. (4)

Strong aspects: Comments to the author: what are the strong aspects of the paper?

In this paper, a new application is proposed. In addition, the paper discusses the different aspects of technology in the field of Programming testing to guarantee programming quality

Weak aspects: Comments to the author: what are the weak aspects of the paper?

Recommended changes: Please indicate any changes that should be made to the paper if accepted. The paper should be more result oriented.

======= Review 2 =======

Relevance and timeliness: Rate the importance and timeliness of the topic addressed in the paper within its area of research.

Acceptable (3)

Technical content and scientific rigour: Rate the technical content of the paper (e.g.: completeness of the Analysis or simulation study, thoroughness of the treatise, accuracy of the models, etc.), its soundness and

scientific rigour.

Valid work but limited contribution. (3)

Novelty and originality: Rate the novelty and originality of the ideas or results presented in the paper. Some interesting ideas and results on a subject well investigated. (3)

Quality of presentation: Rate the paper organization, the clearness of text and figures, the completeness and accuracy of references.

Readable, but revision is needed in some parts. (3)

Strong aspects: Comments to the author: what are the strong aspects of the paper The paper reviews different aspects of the concern

Weak aspects: Comments to the author: what are the weak aspects of the paper?

The future contribution of the paper should be mentioned in the paper

The presentation quality of the paper has to be improved significantly. The details of the author's contribution are too little.

Recommended changes: Recommended changes. Please indicate any changes that should be made to the paper if accepted...

The presentation quality of the paper has to be improved significantly

The detail of simulation/Result is too little.

======= Review 3 =======

Relevance and timeliness: Rate the importance and timeliness of the topic addressed in the paper within its area of research.

Good (4)

Technical content and scientific rigour: Rate the technical content of the paper (e.g.: completeness of the analysis or simulation study, thoroughness of the treatise, accuracy of the models, etc.), its soundness and

scientific rigour.

Marginal work and simple contribution. Some flaws. (2)

Novelty and originality: Rate the novelty and originality of the ideas or results presented in the paper. Minor variations on a well investigated subject. (2)

Quality of presentation: Rate the paper organization, the clearness of text and figures, the completeness and accuracy of references.

Okay. Well Written

Strong aspects: Comments to the author: what are the strong aspects of the paper

In this research, a new scenario of the technology has been introduced; research may be interesting for readers in this area.

Weak aspects: Comments to the author: what are the weak aspects of the paper? Paper should be in IEEE format.

NO ANY MAJOR REVISION

- The presentation should be substantially improved.

- The presentation of the proposed algorithm is too conceptual, and details of how to operate the proposed algorithm in practice should be clearly elaborated.

Thanks, and Regards,

Technical Program Committee Chair 3rd CONIT 2023

conitconf@gmail.com

+91-8767682587

Download the CMT app to access submissions and reviews on the move and receive notifications: https://apps.apple.com/us/app/conference-management-toolkit/id1532488001 https://play.google.com/store/apps/details?id=com.microsoft.research.cmt

To stop receiving conference emails, you can check the 'Do not send me conference email' box from your User Profile.

Microsoft respects your privacy. To learn more, please read our Privacy Statement. Microsoft Corporation

One Microsoft Way Redmond, WA 98052

BALLARI INSITUTE OF TECHNOLOGY AND MANGEMENT,BALLARI DEPARTMENT OF COMPUTER SCIENCE & ENGINEERING

Project CO-PO Mapping ACADEMIC YEAR 2022-23

COURSE OUTCOME(CO’S)

| COURSE OUTCOME STATEMENT |  |
| --- | --- |
| CO-1 | Identify and analyze real-world challenges of digital media manipulation using AI. |
| CO-2 | Automate deepfake detection with machine learning and computer vision techniques. |
| CO-3 | Design a deep learning model using MobileNet and LSTM for fake media detection. |
| CO-4 | Implement the system with Python, TensorFlow, OpenCV, and Streamlit for real-time use. |
| CO-5 | Document and evaluate results through a report highlighting its forensic relevance. |

CO-PO MAPPING

| CO-PO Mapping |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PO-1 | PO-2 | PO-3 | PO-4 | PO-5 | PO-6 | PO-7 | PO-8 | PO-9 | PO-10 | PO-11 | PO-12 | PSO-1 | PSO-2 |
| CO-1 | 2 |  |  |  | 3 |  |  |  |  |  |  |  |  | 3 |
| CO-2 |  | 3 |  | 2 |  |  |  |  |  |  |  |  |  |  |
| CO-3 |  |  | 3 |  | 3 |  |  |  |  |  |  |  |  |  |
| CO-4 |  |  | 3 |  | 3 |  |  |  |  |  |  |  |  | 3 |
| CO-5 |  |  |  |  |  |  |  | 2 | 2 | 2 |  |  |  |  |
| Avg. Mappin g Value | 2 | 3 | 3 | 2 | 3 | 0 | 0 | 3 | 2 | 2 | 2 | 0 | 0 | 3 |

Signature of Guide

PROGRAM OUTCOMES AND PROGRAM SPECIFIC OUTCOMES

PO1. Engineering Knowledge: Apply the knowledge of mathematics, science, engineering fundamentals, and specialization to solve complex problems in computer science and artificial intelligence. PO2. Problem Analysis: Identify, analyze, and review research literature to formulate effective solutions for complex engineering problems using first principles of mathematics, computing, and engineering. PO3. Design/Development of Solutions: Design innovative solutions for real-world digital media challenges, considering ethical, societal, and security implications. PO4. Conduct Investigations of Complex Problems: Utilize research-based knowledge, data analysis, and experimental methods to validate deep learning models and provide accurate conclusions. PO5. Modern Tool Usage: Apply modern AI tools, libraries, and frameworks such as TensorFlow, OpenCV, and Streamlit to develop and evaluate the system effectively. PO6. The Engineer and Society: Assess societal, ethical, and security implications of deepfake content detection, ensuring responsible and lawful use of AI technology. PO7. Environment and Sustainability: Promote awareness of the impact of AI technologies on society and ensure sustainable development in digital ecosystems. PO8. Ethics: Commit to professional ethics by developing responsible AI systems that respect privacy and authenticity in digital communication. PO9. Individual and Team Work: Function effectively as an individual and as a team member in collaborative AI and data science research environments. PO10. Communication: Communicate effectively through technical documentation, research publications, and presentations demonstrating the project’s methodology and findings. PO11. Project Management and Finance: Apply project management principles to plan, execute, and monitor the development and testing of AI-driven detection systems efficiently. PO12. Life-long Learning: Recognize the importance of continuous learning to stay updated with emerging technologies and trends in artificial intelligence and cybersecurity.

PSO1. Understand the architecture and principles of machine learning, neural networks, and deep learning frameworks applied to real-world problems. PSO2. Develop and deploy AI-driven software applications using advanced technologies to address modern challenges in digital media authentication and security.

Project Guide

Project Associates

9

Name: Mr. Mohammed Shafiulla Email: mdshafi@bitm.edu.in Phone no: 8880770638

Name: Ruhe Tarannum

Email: ruhetarannum@gmail.com

USN: 3BR22CS141

Name: Sadiya Tahseen

Email: sadiyatahseen555@gmail.com

USN: 3BR22CS143

Phone no: 8792145056

Phone no: 9449981818

Name: Shahid Ahmed

Email: shahidahmed110604@gmail.com

USN: 3BR22CS150

Phone no: 9036174893

Name: Shaik Farhath Amreen

Email: amreenneema8@gmail.com

USN: 3BR22CS151

Phone no: 8105873758